# HP Insight Management WBEM Smart Array Provider

| Description | The HP Insight Management Web-Based Enterprise Management (WBEM) Management Smart Array provider extends the management capability of referencing profiles by adding the capability to represent HP Management Processor on HP servers. This provider implements the following profiles and installs the necessary files: |  |
| --- | --- | --- |
| Profile Name | Organization | Version |
| HP Smart Array Profile (P00121) | HP WBEM TC | 1.0.0 (P00121) |

For each hardware architecture listed, this provider requires the following distributions

| Requirements | HP Integrity managed servers; SLES 10 and later RLEL 5.0 and later. HP ProLiant managed servers; SLES 11 and later RHEL 5.3 and later. |
| --- | --- |
| Release History | Initial release with HP Insight Management WBEM Providers for Linux v2.0. |

## Setting Up the Provider

| Installing the Provider | There are no special installation instructions for this provider. It is installed by default as part of the HP Insight Management WBEM providers. |
| --- | --- |
| Configuring the Provider | This provider does not accept specific configuration adjustments beyond standard HP Insight Management WBEM support. |

## Using the Provider

Namespaces Supported by the Provider 
This provider returns instances in the root/hpq namespace.

Schema Supported by the Provider

|  | This provider supports the following classes: - SMX_SAArraySystem - SMX_SAArrayController - SMX_SAStorageVolume - SMX_SADiskDrive - SMX_SAStorageExtent - SMX_SADiskDriveFirmware - SMX_SAPhysicalPackage - SMX_SAArrayControllerPhysicalPackage - SMX_SADiskPhysicalPackage - SMX_SAStorageEnclosure - SMX_SAStorageEnclosureProcessor - SMX_SAPortController - SMX_SADAPort - SMX_SASPIPort - SMX_SAFirmware - SMX_SAEnclosureFirmware - SMX_SAPrimordialPool - SMX_SAStoragePool - SMX_SAMediaAccessStatData - SMX_SAStorageSetting - SMX_SADriveCage - SMX_SASCSIProtocolEndpoint - SMX_SASCSIProtocolController - SMX_SARedundancySet - SMX_SAStorageRedundancySet - SMX_SAStorageSpecificCollection - SMX_SADriveCageLocation - SMX_SAArraySystemLocation - SMX_SADiskDriveLocation - SMX_SAArrayControllerLocation - SMX_SAStorageCapabilities - SMX_SAProduct - SMX_SAStorageEnclosureLocation - SMX_SAArraySystemRedundancySet (Association) - SMX_SAStorageVolumeStorageSetting (Association) - SMX_SAPrimordialPoolStorageCapabilities (Association) - SMX_SAStoragePoolStorageCapabilities (Association) - SMX_SAArraySystemStorageEnclosureProcessor (Association) - SMX_SAArraySystemSCSIProtocolController (Association) - SMX_SAArraySystemPortController (Association) - SMX_SAArraySystemDiskDrive (Association) - SMX_SAArraySystemStorageVolume (Association) - SMX_SAArraySystemStorageExtent (Association) - SMX_SAArraySystemSPIPort (Association) - SMX_SAArraySystemDAPort (Association) - SMX_SMX_SAArraySystemDAPort (CIM_SystemDevice) - SMX_SAArraySystemPrimordialPool (Association) - SMX_SAArraySystemStoragePool (Association) - SMX_SAComputerSystemArraySystem (Association) - SMX_SAArraySystemArrayController (Association) - SMX_SAStorageEnclosureStorageEnclosureProcessor (Association) - SMX_SAProductPhysicalPackage (Association) - SMX_SAStorageEnclosureDriveCage (Association) - SMX_SAPhysicalPackageArrayControllerPhysicalPackage (Association) - SMX_SAPhysicalPackageDiskPhysicalPackage (Association) - SMX_SAPhysicalPackageStorageEnclosure (Association) - SMX_SADriveCageDiskPhysicalPackage (Association) - SMX_SAStoragePoolStorageExtent (Association) - SMX_SAPrimordialPoolStorageExtent (Association) - SMX_SAStorageSpecificCollectionArraySystem (Association) - SMX_SAArrayControllerRedundancySet (Association) - SMX_SAStorageRedundancySetStorageExtent (Association) - SMX_SAArraySystemFirmware (Association) - SMX_SASCSIProtocolControllerStorageVolume (Association) - SMX_SAFirmwareArraySystem (Association) - SMX_SAStorageEnclosureProcessorEnclosureFirmware (Association) - SMX_SADiskDriveDiskDriveFirmware (Association) - SMX_SADiskDriveStorageExtent (Association) - SMX_SADiskPhysicalPackageDiskDrive (Association) - SMX_SAStoragePoolStorageVolume (Association) - SMX_SAPrimordialPoolStoragePool (Association) - SMX_SASpareExtentStorageExtent (Association) - SMX_SASpareExtentStorageRedundancySet (Association) - SMX_SAStorageExtentStorageVolume (Association) - SMX_SADAPortSCSIProtocolEndpoint (Association) - SMX_SAStorageGroupHostedCollection (Association) - SMX_SAPhysicalPackageArraySystem (Association) - SMX_SAArrayControllerPhysicalPackageArrayController (Association) - SMX_SADiskDriveMediaAccessStatData (Association) - SMX_SAArrayControllerPhysicalPackageArrayControllerLocation (Association) - SMX_SAPhysicalPackageArraySystemLocation (Association) - SMX_SADriveCageDriveCageLocation (Association) - SMX_SADiskPhysicalPackageDiskDriveLocation (Association) - SMX_SASCSIProtocolEndpointSCSIProtocolController (Association) - SMX_SAStorageEnclosureEnclosureLocation (Association) - SMX_TSASSCSIProtocolEndpoint - SMX_ISASSCSIProtocolEndpoint - SMX_TSASPort - SMX_ISASPort - SMX_ISASPortISASSCSIProtocolEndPoint (Association) - SMX_ArraySystemISASSCSIProtocolEndPoint (Association) - SMX_ArraySystemISASPort (Association) - SMX_PortControllerISASPort (Association) - SMX_ArraySystemTSASSCSIProtocolEndpoint (Association) - SMX_TSASPortTSASSCSIProtocolEndpoint (Association) - SMX_ArraySystemTSASPort (Association) - SMX_DiskDriveTSASSCSIProtocolEndpoint (Association) - SMX_SCSIInitiatorTargetLogicalUnitPath |
| --- | --- |

The tables in the following sections describe the properties of the supported classes. The classes are categorized by the class or superclass that defines the property, the first column is the Property Name (including type and units) and the second column describes how the provider determines the properties implementation. When the Property Implementation value is a number, the number given is the default behavior and the Managed Object Format interpretation is within parenthesis. If other values are returned, a problem is indicated.

Unless otherwise noted, all of the property implementation values given are for HP ProLiant and HP Integrity (cellular and non-cellular) systems. The location related properties and implementation values are determined based on the server type so they may differ.

### SMX_SAArraySystem Class

The SMX_SAArraySystem class implements the HPSA_ArraySystem class and represents the top-level system. There is one instance per HBA.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Controller/Storage System name and <physical location>. Refer to “Physical Location” for more information. For example: “Smart Array in Slot=3” for internal For example: “Smart Array Embedded Controller=7” for embedded controller where 7 is the internal identifier for the embedded controller. |
| CIM_ManagedSystemElement |  |
| OperationalStatus[0] | Overall status of the Array System and attached devices. This is calculated per the algorithm described in the HP Smart Array Profile. 0 (Unknown) 2 (OK) 3 (Degraded) 6 (Error) |
| StatusDescriptions[0] | More info for OperationalStatus result. This provides a short Description to identify who is contributing to non-OK status. For example: Controller Failure Logical Drive Failure Storage Enclosure Failure Controller Non-Failure Problem Physical Drive Non-Failure Problem Logical Drive Non-Failure Problem Storage Enclosure Non-Failure Problem Array is operating properly Array status is unknown |
| CIM_LogicalElement |  |
| CIM_EnabledLogicalElement |  |
| EnabledDefault | 2 (Enabled) |
| EnabledState | 5 (Not Applicable) |
| RequestedState | 12 (Not Applicable) |
| CIM_System |  |
| CreationClassName | SMX_SAArraySystem |
| Name | Controller Serial Number EMBEDDED-<id> will be reported if there is no Serial Number for device is local For example: embedded on motherboard. PCISLOT-<Slot number> will be reported if there is no Serial Number for device is local For example: PCI occupies slot. |
| NameFormat | Other |
| CIM_ComputerSystem |  |
| Dedicated | Dedicated[0]: 3 (Storage) Dedicated[1]: 15 (Block Server) |
| IdentifyingDescriptions | Other Vendor Specific Name |
| OtherIdentifyingInfo | Controller Serial Number EMBEDDED-<id> will be reported if there is no Serial Number for device is local For example: embedded on motherboard. PCISLOT-<Slot number> will be reported if there is no Serial Number for device is local. For example: a PCI occupies slot. |
| Identify() | See HP Smart Array Profile |

### SMX_SAArrayController Class

The SMX_SAArrayController class implements the HPSA_ArrayController class and represents the controller. There is one per HBA and may have multiple controllers.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Controller name and <physical location>. Refer to “Physical Location” for more information. For example: “Smart Array in Slot=3” for internal For example: “Smart Array Embedded Controller=7” for embedded controller where 7 is the internal identifier for the embedded controller. |
| CIM_ManagedSystemElement |  |
| OperationalStatus[0] | Status for Array Controller. 0 (Unknown) 2 (OK) 6 (Error) |
| CIM_LogicalElement |  |
| CIM_EnabledLogicalElement |  |
| EnabledDefault | 2 (Enabled) |
| EnabledState | 5 (Not Applicable) |
| RequestedState | 12 (Not Applicable) |
| CIM_System |  |
| CreationClassName | SMX_SAArrayController |
| Name | Controller Serial Number. EMBEDDED-<id> will be reported if there is no Serial Number. For example: embedded on motherboard. PCISLOT-<Slot number> will be reported if there is no Serial Number and the controller is in a PCI slot Where: <Slot number> is the number of PCI slot. |
| NameFormat | Other |
| CIM_ComputerSystem |  |
| Dedicated | Dedicated[0] 0 (Not Dedicated) |
| HPSA_Arraycontroller |  |
| ADGSupported | See HP Smart Array Profile |
| AcceleratorBackupPowerSource | See HP Smart Array Profile |
| BatteryStatus | See HP Smart Array Profile |
| CacheBoardPresent | See HP Smart Array Profile |
| CacheHasBattery | See HP Smart Array Profile |
| CacheParityReadErrors | See HP Smart Array Profile |
| CacheParityWriteErrors | See HP Smart Array Profile |
| CacheReadPercent | See HP Smart Array Profile |
| CacheSerialNumber | See HP Smart Array Profile |
| CacheSizeTotal | See HP Smart Array Profile |
| CacheState | See HP Smart Array Profile |
| CacheStatus | See HP Smart Array Profile |
| CacheWritePercent | See HP Smart Array Profile |
| ControllerStatus | See HP Smart Array Profile |
| ExpandPriority | See HP Smart Array Profile |
| NumberOfPorts | See HP Smart Array Profile |
| RebuildPriority | See HP Smart Array Profile |
| NumberOfInternalPorts | See HP Smart Array Profile |
| NumberofExternalPorts | See HP Smart Array Profile |

### SMX_SAStorageVolume Class

The SMX_SAStorageVolume class implements the HPSA_StorageVolume class and represents the Logical Volumes configured on the Smart Array.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Logical drive number with raid Description For example: Logical Volume 1 (RAID 1+0) |
| CIM_ManagedSystemElement |  |
| Name | VPD Pg83 identifier for the volume. |
| CIM_LogicalElement |  |
| CIM_EnabledLogicalElement |  |
| EnabledDefault | 2 (Enabled) |
| EnabledState | 5 (Not Applicable) |
| RequestedState | 12 (Not Applicable) |
| CIM_LogicalDevice |  |
| CreationClassName | SMX_SAStorageVolume |
| DeviceID | Opaque Key |
| SystemCreationClassName | SMX_SAArraySystem |
| SystemName | SMX_SAArraySystem.Name |
| CIM_StorageExtent |  |
| ConsumableBlocks | 0 |
| BlockSize | Block Size in bytes |
| DataRedundancy | 1 (RAID 0, RAID 5, RAID 6), RAID 50, RAID 60 2 (RAID 1+0) |
| DeltaReservation | 0 |
| ExtentStatus[0] | This will indicate extended status for the volume. 2 (None) 11 (Rebuild – Volume is currently rebuilding data) |
| IsBasedOnUnderlyingRedundancy | False |
| NoSinglePointOfFailure | True – RAID 1 or above False – RAID 0 |
| NumberOfBlocks | Total Number of blocks on the volume. |
| PackageRedundancy | 0 (RAID 0) 1 (RAID 1+0, RAID 5) 2 (RAID 6), RAID 50, RAID 60 |
| CIM_Storagevolume |  |
| NameFormat | 2 (VPD83NAA6) |
| HPSA_Storagevolume |  |
| Accelerator | See HP Smart Array Profile |
| Deleteable | See HP Smart Array Profile |
| FaultTolerance | See HP Smart Array Profile |
| OperationalStatus[0, 1] | Volume status. DMTF defined value will be populated in index[0] and extended status will be populated in index[1] per HP Smart Array Profile. |
| OSName | Corresponding OS name for this volume. |
| StripeSize | See HP Smart Array Profile |
| Identify() | See HP Smart Array Profile |

### SMX_SADiskDrive Class

The SMX_SADiskDrive class implements the HPSA_DiskDrive class and represents the hard drives connected to Smart Array controllers.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Disk location For example: Port: 1E Box 1 Bay: 5 |
| CIM_ManagedSystemElement |  |
| Name | Drive Serial Number |
| OperationalStatus[0] | Drive status, DMTF defined value is populated in index (0) and extended status is populated in index(1) per HP Smart Array Profile |
| CIM_LogicalElement |  |
| CIM_EnabledLogicalElement |  |
| EnabledDefault | 2 (Enabled) |
| EnabledState | 5 (Not Applicable) |
| RequestedState | 12 (Not Applicable) |
| CIM_LogicalDevice |  |
| CreationClassName | SMX_SADiskDrive |
| DeviceID | Opaque value |
| SystemCreationClassName | SMX_SAArraySystem |
| SystemName | SMX_SAArraySystem.Name |
| TotalPowerOnHours | Number of service hours logged for the drive |
| CIM_MediaAccessDevice |  |
| CIM_DiskDrive |  |
| HPSA_DiskDrive |  |
| DriveConfiguration | See HP Smart Array Profile |
| DriveHasMNPData | See HP Smart Array Profile |
| DriveInterface | See HP Smart Array Profile |
| DrivePlacement | See HP Smart Array Profile |
| DriveRotationalSpeed | See HP Smart Array Profile |
| NegotiatedDataWidth | See HP Smart Array Profile |
| NegotiatedSpeed | See HP Smart Array Profile |
| Identify() | See HP Smart Array Profile |

### SMX_SAStorageExtent Class

The SMX_SAStorageExtent class implements the HPSA_StorageExtent class and represents the consumable storage that is available on a disk drive. There is one instance per disk drive.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Disk location For example: Port: 1E Box 1 Bay: 5 |
| CIM_ManagedSystemElement |  |
| Name | Serial number of drive |
| OperationalStatus[0] | Operational Status for the disk drive 0 (Unknown) 2 (OK) 5 (Predictive Failure) 6 (Error) |
| CIM_LogicalElement |  |
| CIM_EnabledLogicalElement |  |
| EnabledDefault | 2 (Enabled) |
| EnabledState | 5 (Not Applicable) |
| RequestedState | 12 (Not Applicable) |
| CIM_LogicalDevice |  |
| CreationClassName | SMX_SAStorageExtent |
| DeviceID | Opaque ID |
| SystemCreationClassName | SMX_SAArraySystem |
| SystemName | SMX_SAArraySystem.Name |
| CIM_StorageExtent |  |
| BlockSize | Block size in bytes |
| ConsumableBlocks | Total number of blocks on disk drive |
| ExtentStatus[0] | This will indicate extended status for the disk drive Possible values: 2 (None) 11 (Rebuild – disk drive is currently rebuilding data) |
| NumberOfBlocks | Total number of blocks on disk drive |
| Primordial | Set to True |
| TotalPowerOnHours | Total service hours reported disk drive |
| HPSA_StorageExtent |  |

### SMX_SADiskDriveFirmware Class

The SMX_SADiskDriveFirmware class implements the HPSA_DiskDriveFirmware class and represents the firmware installed on a disk drive. There is one instance per disk drive.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | TargetType value |
| CIM_ManagedSystemElement |  |
| Name | Disk Drive Firmware |
| CIM_LogicalElement |  |
| Caption | Disk Drive Firmware |
| Description | Disk Drive Firmware |
| CIM_SoftwareIdentity |  |
| ClassificationDescriptions[0] | Disk Drive Firmware |
| Classifications[0] | 10 (Firmware) |
| InstanceID | Opaque key |
| Manufacturer | Disk drive vendor string |
| IdentityInfoType | [0]CIM:SoftwareFamily [1]HPQ:SoftwareCategory |
| IdentityInfoValue | [0]HPQ:<TargetType value> [1]Storage Device |
| VersionString | Drive firmware as reported from SCSI identify command |
| HPSA_DiskDriveFirmware |  |

### SMX_SAPhysicalPackage Class

The SMX_SAPhysicalPackage class implements the HPSA_PhysicalPackage class and represents the overall physical package for the array system as a whole. There is one instance per HBA or Storage Subsystem.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Controller/Storage System name and <physical location>. Refer to “Physical Location” for more information. |
| Description | Host Based RAID Controller ( For Internal Controller) |
| CIM_ManagedSystemElement |  |
| Name | See Serial number implementation |
| CIM_PhysicalElement |  |
| CreationClassName | SMX_SAPhysicalPackage |
| Manufacturer | Controller vendor string |
| Model | Controller model string |
| PartNumber | N/A – Not Available for this implementation |
| SerialNumber | Controller Serial number for HBA’s |
| Tag | Opaque Key |
| Version | N/A – Not Available for this implementation |
| CIM_PhysicalPackage |  |
| RemovalConditions | 2 (Not Applicable) |
| HPSA_PhysicalPackage |  |

### SMX_SAArrayControllerPhysicalPackage Class

The SMX_SAArrayControllerPhysicalPackage class implements the HPSA_ArrayControllerPhysicalPackage class and represents the physical aspects of any Smart Array controller. There is one instance per HBA.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Controller name and <physical location>. Refer to “Physical Location” for more information. For example: “Smart Array in Slot=3” for internal For example: “Smart Array Embedded Controller=7” for embedded controller where 7 is the internal identifier for the embedded controller. |
| CIM_ManagedSystemElement |  |
| Name | Controller Serial number |
| CIM_ PhysicalElement |  |
| CreationClassName | SMX_SAArrayControllerPhysicalPackage |
| Manufacturer | Controller vendor string |
| Model | Controller model string |
| PartNumber | N/A – Not Available for this implementation |
| SerialNumber | Controller Serial number |
| Tag | Opaque Key |
| Version | N/A – Not Available for this implementation |
| CIM_PhysicalPackage |  |
| RemovalConditions | 2 (Not Applicable) |
| HPSA_ ArrayControllerPhysicalPackage |  |

### SMX_SADiskPhysicalPackage Class

The SMX_SADiskPhysicalPackage class represents the physical aspects of the disk drive. There is one instance per disk drive.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Disk location For example: Port: 1E Box 1 Bay: 5 |
| CIM_ManagedSystemElement |  |
| Name | Disk location For example: Port: 1E Box 1 Bay: 5 |
| CIM_ PhysicalElement |  |
| CreationClassName | SMX_SADiskPhysicalPackage |
| Manufacturer | Drive vendor string |
| Model | Drive model string |
| PartNumber | N/A – Not Available in this implementation |
| SerialNumber | Drive serial number |
| Tag | Opaque Key |
| Version | N/A – Not Available in this implementation |
| CIM_PhysicalPackage |  |
| HotSwappable | TRUE - Drive is hot swappable FALSE – Drive is not hot swappable |
| RemovalConditions | 2 (Not Applicable) |
| HPSA_ DiskPhysicalPackage |  |

### SMX_SAStorageEnclosure Class

The SMX_SAStorageEnclosure class implements the HPSA_StorageEnclosureclass and represents the physical chassis for an internal or externally attached enclosure.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Port: <m> Box: <n> Where: <m> is the port and <n> is the box number. |
| CIM_ManagedSystemElement |  |
| Name | Port: <m> Box: <n> Where: <m> is the port and <n> is the box number. |
| CIM_PhysicalElement |  |
| CreationClassName | SMX_SAStorageEnclosure |
| Manufacturer | Enclosure vendor string For example: HP For example: N/A if not available |
| Model | Enclosure model string For example: PROLIANT 6L6I For example: N/A if not available |
| PartNumber | N/A – Not Available in this implementation |
| SerialNumber | Enclosure Serial Number. N/A will be reported if there is no serial number (For example: internal enclosures). |
| Tag | Opaque key |
| Version | N/A – Not Available in this implementation |
| CIM_PhysicalPackage |  |
| RemovalConditions | 2 (Not Applicable) |
| CIM_PhysicalFrame |  |
| CIM_Chassis |  |
| HP_StorageEnclosure |  |
| Location | See HP Storage Enclosure Profile |
| HPSA_StorageEnclosure |  |

### SMX_SAStorageEnclosureProcessor Class

The SMX_SAStorageEnclosureProcessor class represents the management processor within a Storage Enclosure. This instance contains the operational status for all managed components in the enclosure if applicable.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Port: <m> Box: <n> Where: <m> is the port and <n> is the box number. |
| CIM_ManagedSystemElement |  |
| OperationalStatus[0,1, 2, 3 ] | Operational status for enclosure and associated power supplies, fans and sensors. The use of OperationalStatus is described in the HP Storage Enclosure Profile. Index values are populated as follows. Index[0] – Enclosure status: 0 (Unknown) 2 (OK) 3 (Degraded) 6 (Error) Index[1] – Fan status (See Profile) Index[2] – Power Supply status (See Profile) Index[3] – Temp Sensor Status (See Profile) Note: Operational status for Fan, Power and Temp Sensor will be not detectedfor internal storage enclosure(s) of the local controller(s) For example: PCI or Embedded. |
| CIM_LogicalElement |  |
| CIM_EnabledLogicalElement |  |
| EnabledDefault | 2 (Enabled) |
| EnabledState | 5 (Not Applicable) |
| RequestedState | 12 (Not Applicable) |
| CIM_System |  |
| CreationClassName | SMX_SAStorageEnclosureProcessor |
| Name | Opaque Key |
| NameFormat | Other |
| CIM_ComputerSystem |  |
| Dedicated[0] | 2 (Other) |
| IdentifyingDescriptions[0] | Other Vendor Specific Name |
| OtherDedicatedDescriptions[0] | SDE - indicates Storage Device Enclosure |
| OtherIdentifyingInfo[0] | Port Number:BoxNumber enclosure is attached to. For example: 1E:1 |
| HPSA_StorageEnclosureProcessor |  |

### SMX_SAPortController Class

The SMX_SAPortController class represents the HBA in the host system. There is one instance per HBA.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Controller name and <physical location>. Refer to “Physical Location” for more information. For example: “Smart Array in Slot=3” for internal For example: “Smart Array Embedded Controller=7” for embedded controller where 7 is the internal identifier for the embedded controller. |
| CIM_ManagedSystemElement |  |
| CIM_LogicalElement |  |
| CIM_EnabledLogicalElement |  |
| EnabledDefault | 2 (Enabled) |
| EnabledState | 5 (Not Applicable) |
| RequestedState | 12 (Not Applicable) |
| CIM_LogicalDevice |  |
| CreationClassName | SMX_SAPortController |
| DeviceID | Controller serial number |
| SystemCreationClassName | SMX_SAArraySystem |
| SystemName | SMX_SAArraySystem.Name |
| CIM_Controller |  |
| CIM_PortController |  |
| ControllerType | 1 (Other) |
| OtherControllerType | SAS/SATA – SAS/SATA HBA |
| HPSA_PortController |  |

### SMX_SADAPort Class

The SMX_SADAPort class implements the HPSA_DAPort class and represents the front-end port for an HBA in the host system. Direct attached implies that the array controller is attached to a local bus on the host, such as PCI.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Controller name and <physical location>. Refer to “Physical Location” for more information. For example: “Smart Array in Slot=3” for internal For example: “Smart Array Embedded Controller=7” for embedded controller where 7 is the internal identifier for the embedded controller. |
| CIM_ManagedSystemElement |  |
| OperationalStatus[0] | Port status 0 (Unknown) 2 (OK) 6 (Error) |
| CIM_LogicalElement |  |
| CIM_EnabledLogicalElement |  |
| EnabledDefault | 2 (Enabled) |
| EnabledState | 5 (Not Applicable) |
| RequestedState | 12 (Not Applicable) |
| CIM_LogicalDevice |  |
| CreationClassName | SMX_SADAPort |
| DeviceID | Opaque key |
| SystemCreationClassName | SMX_SAArraySystem |
| SystemName | SMX_SAArraySystem.Name |
| CIM_LogicalPort |  |
| UsageRestriction | 2 (Front-End Only) |
| CIM_DAPort |  |
| HPSA_DAPort |  |

### SMX_SAFirmware Class

The SMX_SAFirmware class implements the HPSA_Firmware class and represents the firmware installed on a Smart Array controller.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | TargetType value |
| CIM_ManagedSystemElement |  |
| Name | Array Controller Firmware |
| CIM_LogicalElement |  |
| Caption | Array Controller Firmware |
| Description | Array Controller Firmware |
| CIM_SoftwareIdentity |  |
| ClassificationDescriptions[0] | Array Controller Firmware |
| Classifications[0] | 10 (Firmware) |
| InstanceID | Opaque key |
| Manufacturer | Controller Vendor string |
| IdentityInfoType | [0]CIM:SoftwareFamily [1]HPQ:SoftwareCategory |
| IdentityInfoValue | [0]HPQ:<TargetType value> [1]Storage Controller |
| VersionString | Controller firmware version string |
| HPSA_Firmware |  |

### SMX_SAEnclosureFirmware Class

The SMX_SAEnclosureFirmware class implements the HPSA_EnclosureFirmware class and represents the firmware installed on a Smart Array Enclosure.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | TargetType value |
| CIM_ManagedSystemElement |  |
| Name | Storage Enclosure Processor Firmware |
| CIM_LogicalElement |  |
| Caption | Storage Enclosure Processor Firmware |
| Description | Storage Enclosure Processor Firmware |
| CIM_SoftwareIdentity |  |
| ClassificationDescriptions[0] | Storage Enclosure Processor Firmware |
| Classifications[0] | 10 (Firmware) |
| InstanceID | Opaque key |
| Manufacturer | Enclosure Vendor string |
| IdentityInfoType | [0]CIM:SoftwareFamily [1]HPQ:SoftwareCategory |
| IdentityInfoValue | [0]HPQ:<TargetType value> [1]StorageDevice |
| VersionString | Enclosure firmware string |
| HPSA_EnclosureFirmware |  |

### SMX_SAPrimordialPool Class

The SMX_SAPrimordialPool class implements the HPSA_PrimordialPool class and represents the total raw storage managed by the array controller. There is one instance per array system.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Controller name and <physical location>. Refer to “Physical Location” for more information. For example: “Smart Array in Slot=3” for internal For example: “Smart Array Embedded Controller=7” for embedded controller where 7 is the internal identifier for the embedded controller. |
| CIM_ManagedSystemElement |  |
| CIM_LogicalElement |  |
| CIM_StoragePool |  |
| InstanceID | Opaque key |
| PoolID | Controller serial number |
| Primordial | Always set to TRUE |
| RemainingManagedSpace | Remaining raw space on controller. This is calculated by summing the remaining space on all drives attached to the controller. |
| TotalManagedSpace | Total raw space available to controller. This is calculated by summing the total space on all drives connected to this controller. |
| HPSA_PrimordialPool |  |

### SMX_SAStoragePool Class

The SMX_SAStoragePool class implements the HPSA_StoragePool class and represents any concrete storage pools created on an array controller. There is one instance per array system.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Controller name and <physical location>. Refer to “Physical Location” for more information. For example: “Smart Array in Slot=3” for internal For example: “Smart Array Embedded Controller=7” for embedded controller where 7 is the internal identifier for the embedded controller. |
| CIM_ManagedSystemElement |  |
| CIM_LogicalElement |  |
| CIM_StoragePool |  |
| InstanceID | Opaque key |
| PoolID | Controller serial number: <n> where n is the array number as supplied from API. |
| Primordial | Always set to FALSE |
| RemainingManagedSpace | Remaining raw space in current pool |
| TotalManagedSpace | Total raw space managed by this pool |
| HPSA_StoragePool |  |

### SMX_SAMediaAccessStatData Class

The SMX_SAMediaAccessStatData class implements the HPSA_MediaAccessStatData and represents the Monitoring and Performance data counters on HP Disk Drives.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| CIM_StatisticalData |  |
| InstanceID | Opaque key |
| SampleInterval | 0 |
| CIM_MediaAccessStatData |  |
| HPSA_MediaAccessStatData |  |
| AbortedCommands | See HP Smart Array Profile |
| BadTargetErrors | See HP Smart Array Profile |
| DRQTimeouts | See HP Smart Array Profile |
| ECCRecoveredReadErrors | See HP Smart Array Profile |
| ElementName | Disk location For example: Port: 1E Box 1 Bay: 5 |
| FailedReadRecovers | See HP Smart Array Profile |
| FailedWriteRecovers | See HP Smart Array Profile |
| FormatErrors | See HP Smart Array Profile |
| HardReadErrors | See HP Smart Array Profile |
| HardwareErrors | See HP Smart Array Profile |
| HardWriteErrors | See HP Smart Array Profile |
| HotPlugCount | See HP Smart Array Profile |
| MediaFailures | See HP Smart Array Profile |
| NotReadyErrors | See HP Smart Array Profile |
| OtherTimeOuts | See HP Smart Array Profile |
| SCSIBusFaults | See HP Smart Array Profile |
| SectorsRead | See HP Smart Array Profile |
| SectorsWritten | See HP Smart Array Profile |
| SeekCount | See HP Smart Array Profile |
| SeekErrors | See HP Smart Array Profile |
| ServiceHours | See HP Smart Array Profile |
| SpinUpErrors | See HP Smart Array Profile |
| RetryRecoveredReadErrors | See HP Smart Array Profile |
| RetryRecoveredWriteErrors | See HP Smart Array Profile |

### SMX_SAStorageSetting Class

The SMX_SAStorageSetting class implements the HPSA_StorageSetting class and represents the current raid configuration for a storage volume. There is one instance per storage volume.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | RAID <n> Where: <n> is the RAID Description. For example: RAID 0 For example: RAID 1+0 For example: RAID 5 For example: RAID 6 |
| CIM_SettingData |  |
| InstanceID | Opaque key |
| CIM_StorageSetting |  |
| ChangeableType | 0 (Not Changeable) |
| DataRedundancyGoal | 1 (RAID 0, RAID 4, RAID 5, RAID 6, RAID 50, RAID 60) 2 (RAID 1, RAID 1+0, RAID 5+1) |
| DataRedundancyMax | 1 (RAID 0, RAID 4, RAID 5, RAID 6, RAID 50, RAID 60) 2 (RAID 1, RAID 1+0, RAID 5+1) |
| DataRedundancyMin | 1 (RAID 0, RAID 4, RAID 5, RAID 6, RAID 50, RAID 60) 2 (RAID 1, RAID 1+0, RAID 5+1) |
| DeltaReservationGoal | 0 |
| DeltaReservationMax | 0 |
| DeltaReservationMin | 0 |
| NoSinglePointOfFailure | False – RAID 0 True – RAID 1+0, RAID 5, RAID 6, RAID 50, RAID 60 |
| PackageRedundancyGoal | 0 (RAID 0) 1 (RAID 1, RAID 1+0, RAID 4, RAID 5, RAID 6,) 2 (RAID 6, RAID 50, RAID 60) |
| PackageRedundancyMax | 0 (RAID 0) 1 (RAID 1+0, RAID 5, RAID 5+1) 2 (RAID 6, RAID 50, RAID 60) |
| PackageRedundancyMin | 0 (RAID 0) 1 (RAID 1, RAID 1+0, RAID 4, RAID 5, RAID 5+1) 2 (RAID 6, RAID 50, RAID 60) |
| HPSA_StorageSetting |  |

### SMX_SADriveCage Class

The SMX_SADriveCage class implements the HPSA_DriveCage class and represents the drive cage assembly and back plane within a storage enclosure. There may be multiple instances within a duplexed enclosure.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Port <n> Where: <n> is the name of the port this drive cage is connected. For example: Port J3 |
| CIM_ManagedSystemElement |  |
| Name | Serial Number of drive cage N/A – when serial number is not available |
| CIM_PhysicalElement |  |
| CreationClassName | SMX_SADriveCage |
| Manufacturer | Drive Cage Vendor string |
| Model | Drive Cage Model string |
| PartNumber | N/A – Not available in this implementation |
| Tag | Opaque key |
| SerialNumber | Enclosure Serial number |
| Version | N/A – Not available in this implementation |
| CIM_PhysicalPackage |  |
| RemovalConditions | 2 (Not Applicable) |
| HP_DriveCage |  |
| BackplaneSpeed | See HP Smart Array profile |
| DriveBays | See HP Smart Array profile |
| DuplexLocation | See HP Smart Array profile |
| HPSA_DriveCage |  |

### SMX_SASCSIProtocolEndpoint Class

The SMX_SASCSIProtocolEndpoint class implements the HPSA_SCSIProtocolEndpoint class and represents an access point to storage resources.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Controller name and <physical location>. Refer to “Physical Location” for more information. For example: “Smart Array in Slot=3” for internal For example: “Smart Array Embedded Controller=7” for embedded controller where 7 is the internal identifier for the embedded controller. |
| CIM_ManagedSystemElement |  |
| CIM_LogicalElement |  |
| CIM_EnabledLogicalElement |  |
| EnabledDefault | 2 (Enabled) |
| RequestedState | 12 (Not Applicable) |
| CIM_ServiceAccessPoint |  |
| CreationClassName | SMX_SASCSIProtocolEndpoint |
| Name | HBA Serial Number |
| SystemCreationClassName | SMX_SAArraySystem |
| SystemName | SMX_SAArraySystem.Name |
| CIM_ProtocolEndpoint |  |
| ProtocolIFType | 1 (Other) |
| OtherTypeDescription | SCSI |
| CIM_SCSIProtocolEndpoint |  |
| ConnectionType | 1 (Other) |
| OtherConnectionType | PCI |
| Role | 0 (Unknown) 2 (Initiator) 3 (Target) 4 (Both Initiator and Target) |
| HPSA_SCSIProtocolEndpoint |  |

### SMX_SASCSIProtocolController Class

The SMX_SASCSIProtocolController class implements the HPSA_SCSIProtocolController class and represents a view a SCSI resources on the array controller.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Controller name and <physical location>. Refer to “Physical Location” for more information. For example: “Smart Array in Slot=3” for internal For example: “Smart Array Embedded Controller=7” for embedded controller where 7 is the internal identifier for the embedded controller. |
| CIM_ManagedSystemElement |  |
| OperationalStatus[0] | Operational status of the controller 0 (Unknown) 2 (OK) 6 (Error) |
| CIM_LogicalElement |  |
| CIM_EnabledLogicalElement |  |
| EnabledDefault | 2 (Enabled) |
| EnabledState | 5 (Not Applicable) |
| RequestedState | 12 (Not Applicable) |
| CIM_LogicalDevice |  |
| CreationClassName | SMX_SASCSIProtocolController |
| DeviceID | Opaque key |
| SystemCreationClassName | SMX_SAArraySystem |
| SystemName | SMX_SAArraySystem.Name |
| CIM_ProtocolController |  |
| MaxUnitsControlled | 0 |
| CIM_ SCSIProtocolController |  |
| HPSA_ SCSIProtocolController |  |

### SMX_SARedundancySet Class

The SMX_SARedundancySet class implements the HPSA_RedundancySet class and represents controller redundancy if there is more than one controller in a redundant configuration. For example: an MSA500 with two controllers.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Storage System name and location |
| CIM_Collection |  |
| CIM_SystemSpecificCollection |  |
| InstanceID | Opaque key |
| CIM_RedundancySet |  |
| LoadBalanceAlgorithm | 2 (No Load Balancing) |
| RedundancyStatus | 2 (Fully Redundant) 4 (Redundancy Lost) |
| TypeOfSet[0] | 4 (Sparing) |
| HPSA_RedundancySet |  |

### SMX_SAStorageRedundancySet Class

The SMX_SAStorageRedundancySet class implements the HPSA_StorageRedundancySet class and represents disk sparing aspect of a RAID configuration. There is an instance for every logical drive that has spare drives configured.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Storage Redundancy Set |
| CIM_Collection |  |
| CIM_SystemSpecificCollection |  |
| InstanceID | Opaque key |
| CIM_RedundancySet |  |
| LoadBalanceAlgorithm | 2 (No Load Balancing) |
| MaxNumberSupported | 0 |
| MinNumberNeeded | 0 |
| RedundancyStatus | 2 (Fully Redundant) 4 (Redundancy Lost) |
| TypeOfSet[0] | 5 (Limited Sparing) |
| CIM_StorageRedundancySet |  |
| HPSA_StorageRedundancySet |  |

### SMX_SAStorageSpecificCollection Class

The SMX_SAStorageSpecificCollection class implements the HPSA_StorageSpecificCollection class and represents a collection of smart array controllers that have been grouped together to report consolidated status.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Storage Array Consolidated Status |
| Description | Storage Array Consolidated Status |
| ElementName | Storage Array Consolidated Status |
| CIM_Collection |  |
| CIM_SystemSpecificCollection |  |
| InstanceID | Opaque key |
| HP_GroupSystemSpecificCollection |  |
| GroupOperationalStatus[0] | Consolidated status for all array controllers. Refer to section 9.10 of the HP Smart Array profile for details on how this is calculated and status example. |
| GroupStatusDescriptions[0] | Description of GroupOperationStatus |
| HPSA_ StorageSpecificCollection |  |

### SMX_SADriveCageLocation Class

The SMX_SADriveCageLocation class implements the HPSA_DriveCageLocation class and represents a location string for the drive cage.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Drive Cage |
| Description | Drive Cage |
| ElementName | Drive Cage |
| CIM_Location |  |
| Name | Opaque key |
| PhysicalPosition | Opaque key |
| HP_Location |  |
| ElementLocationTag | Duplex Top, Duplex Bottom, Non Duplex |
| ElementLocationTagDesc | 22 (Drive Cage) |
| LocationInformation[0] | HBA Port identifier For example: J3 For example: 2I For example: 1E |
| LocationInfoDesc[0] | Port |
| HPSA_DriveCageLocation |  |

### SMX_SAArraySystemLocation Class

The SMX_SAArraySystemLocation class implements the HPSA_ArraySystemLocation class and represents a location string for the array system.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Smart Array in <physical location> Refer to “Physical Location” for more information. |
| Description | Same as Caption. |
| ElementName | Same as Caption. |
| CIM_Location |  |
| Name | Opaque key |
| PhysicalPosition | Opaque key |
| HP_Location |  |
| ElementLocationTag | PCI Slot Number – when ‘Adapter’ is specified in ElementLocationTagDesc |
| ElementLocationTagDesc | 23 (Adapter) – for internal HBA’s |
| LocationInformation[0] | Array of text values representing the location components that describe this Ethernet Controller Physical location. LocationInfoDesc[0] per LocationInfornation[0]. Refer to the LocationInfoDescription array to determine what type of location information is represented. |
| LocationInfoDesc[0] | 3 (PCI Slot) – for internal HBA’s |
| HPSA_ArraySystemLocation |  |

### SMX_SADiskDriveLocation Class

The SMX_SADiskDriveLocation class implements the HPSA_DiskDriveLocation class and represents a location string for the disk drive.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Controller name, Drive Type and Disk Location For example: Smart Array E500 Physical Drive – Port: 1E Box 1 Bay: 5 |
| Description | Controller name, Drive Type and Disk Location For example: Smart Array E500 Physical Drive - Port: 1E Box 1 Bay: 5 |
| ElementName | Controller name, Drive Type and Disk Location For example: Smart Array E500 Physical Drive - Port: 1E Box 1 Bay: 5 |
| CIM_Location |  |
| Name | Opaque key |
| PhysicalPosition | Opaque key |
| HP_Location |  |
| ElementLocationTag | Bay Number of Disk Drive - When ‘Bay’ is specified in ElementLocationTagDesc. |
| ElementLocationTagDesc | 20 (Bay)– when firmware supports Bay numbering |
| HPSA_DiskDriveLocation |  |

### SMX_SAArrayControllerLocation Class

The SMX_SAArrayControllerLocation class implements the HPSA_ArrayControllerLocation class and represents a location string for the array controller.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Smart Array in <physical location> Refer to “Physical Location” for more information. |
| Description | Same as Caption. |
| ElementName | Same as Caption. |
| CIM_Location |  |
| Name | Opaque key |
| PhysicalPosition | Opaque key |
| HP_Location |  |
| ElementLocationTag | For Internal HBA’s: PCI Slot Number For example: 3 – for adapter in PCI Slot 3 For example: 0 – for Embedded adapter |
| ElementLocationTagDesc | 23 (Adapter) – for Internal HBA’s |
| LocationInformation[0] | Array of text values representing the location components that describe this Ethernet Controller Physical location. LocationInfoDesc[0] per LocationInfornation[0]. Refer to the LocationInfoDescription array to determine what type of location information is represented. |
| LocationInfoDesc[0] | 1(MezzanineSlot)—for internal HBA on a Mezzanine Card 3 (PCI Slot) – for internal HBA in a PCI slot 17(Embedded Device) – for internal HBA embedded on Motherboard |
| HPSA_ArrayControllerLocation |  |

### SMX_SAStorageCapabilities Class

The SMX_SAStorageCapabilities class implements the HPSA_StorageCapabilities class and represents the possible Capabilities of a storage pool.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | For Primodial Pools: Controller name and <physical location>. Refer to “Physical Location” for more information. For example: “Smart Array in Slot=3” for internal For example: “Smart Array Embedded Controller=7” for embedded controller where 7 is the internal identifier for the embedded controller. For Concrete Pools: Controller name and <physical location>, Array Identifier For example: Smart Array in Slot=3:0 |
| CIM_Capabilities |  |
| InstanceID | Key: Opaque key |
| CIM_StorageCapabilities |  |
| DataRedundancyDefault | 1 (When number of drives is an odd number) 2 (When number of drives is an even number – For example: Pool can be configured for mirroring) |
| DataRedundancyMax | 1 (When number of drives is an odd number) 2 (When number of drives is an even number – For example: Pool can be configured for mirroring) |
| DataRedundancyMin | 1 |
| DeltaReservationDefault | 0 |
| DeltaReservationMax | 0 |
| DeltaReservationMin | 0 |
| ElementType | 5 (Storage Pool) |
| NoSinglePointOfFailure | True - When number of drives > 1 (For example: Pool can be configured for redundancy) False – When number of drives <= 1 |
| NoSinglePointOfFailureDefault | True - When number of drives > 1 (For example: Pool can be configured for redundancy) False – When number of drives <= 1 |
| PackageRedundancyDefault | 0 (When number of drives <= 1) 1 (When number of drives = 2 or 3) 2 (When number of drives > 3) |
| PackageRedundancyMax | 0 (When number of drives <= 1) 1 (When number of drives = 2 or 3) 2 (When number of drives > 3) |
| PackageRedundancyMin | 0 |
| HPSA_StorageCapabilities |  |

### SMX_SAProduct Class

The SMX_SAProduct class implements the HPSA_Product class and represents the top level marketing information for the array system.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Controller name and <physical location>. Refer to “Physical Location” for more information. For example: “Smart Array in Slot=3” for internal For example: “Smart Array Embedded Controller=7” for embedded controller where 7 is the internal identifier for the embedded controller. |
| CIM_Product |  |
| IdentifyingNumber | Opaque key |
| Name | String representing the Marketing name. |
| Vendor | HP |
| Version | Firmware version of HBA or Storage System. |
| HPSA_Product |  |

### SMX_SAStorageEnclosureLocation Class

The SMX_SAStorageEnclosureLocation class implements the HPSA_StorageEnclosureLocation class and represents a location string for the enclosure.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| ElementName | Enclosure location Port: <m> Box: <n> Where: <m> is the port and <n> is the box number. |
| Caption | Enclosure location Port: <m> Box: <n> Where: <m> is the port and <n> is the box number. |
| Description | Enclosure location Port: <m> Box: <n> Where: <m> is the port and <n> is the box number. |
| Name | Key |
| PhysicalPosition | Key |
| ElementLocationTag | Unknown for Undetermined External for external enclosures Internal for drive cages located within the host chassis |
| ElementLocationTagDesc | 19 (Box) |
| HPSA_StorageEnclosureLocation |  |

### SMX_SAArraySystemRedundancySet (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| SameElement | SMX_SARedundancySet |
| SystemElement | SMX_SAArraySystem |

### SMX_SAStorageVolumeStorageSetting (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| ManagedElement | SMX_SAStorageVolume |
| SettingData | SMX_SAStorageSetting |
| IsCurrent | 1 (Set to Is Current) |
| IsDefault | 1 (Set to Is Default) |

### SMX_SAPrimordialPoolStorageCapabilities (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Capabilities | SMX_SAStorageCapabilities |
| ManagedElement | SMX_SAPrimordialPool |

### SMX_SAStoragePoolStorageCapabilities (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Capabilities | SMX_SAStorageCapabilities |
| ManagedElement | SMX_SAStoragePool |

### SMX_SAArraySystemStorageEnclosureProcessor (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | SMX_SAArraySystem |
| PartComponent | SMX_SAStorageEnclosureProcessor |

### SMX_SAArraySystemSCSIProtocolController (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | SMX_SAArraySystem |
| PartComponent | SMX_SASCSIProtocolController |

### SMX_SAArraySystemPortController (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| SameElement | SMX_SAArraySystem |
| SystemElement | SMX_SAPortController |

### SMX_SAArraySystemDiskDrive (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | SMX_SAArraySystem |
| PartComponent | SMX_SADiskDrive |

### SMX_SAArraySystemStorageVolume (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | SMX_SAArraySystem |
| PartComponent | SMX_SAStorageVolume |

### SMX_SAArraySystemStorageExtent (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | SMX_SAArraySystem |
| PartComponent | SMX_SAStorageExtent |

### SMX_SAArraySystemSPIPort (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | SMX_SAArraySystem |
| PartComponent | SMX_SASPIPort |

### SMX_SAArraySystemDAPort (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | SMX_SAArraySystem |
| PartComponent | SMX_SADAPort |

### SMX_SAArraySystemPrimordialPool (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | SMX_SAArraySystem |
| PartComponent | SMX_SAPrimordialPool |

### SMX_SAArraySystemStoragePool (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | SMX_SAArraySystem |
| PartComponent | SMX_SAStoragePool |

### SMX_SAComputerSystemArraySystem (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | HP_ComputerSystem |
| PartComponent | SMX_SAArraySystem |

### SMX_SAArraySystemArrayController (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | SMX_SAArraySystem |
| PartComponent | SMX_SAArrayController |

### SMX_SAStorageEnclosureStorageEnclosureProcessor (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | SMX_SAStorageEnclosure |
| Dependent | SMX_SAStorageEnclosureProcessor |

### SMX_SAProductPhysicalPackage (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | SMX_SAProduct |
| PartComponent | SMX_SAPhysicalPackage |

### SMX_SAStorageEnclosureDriveCage (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | SMX_SAStorageEnclosure |
| PartComponent | SMX_SADriveCage |

### SMX_SAPhysicalPackageArrayControllerPhysicalPackage (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | SMX_SAPhysicalPackage |
| PartComponent | SMX_SAArrayControllerPhysicalPackage |

### SMX_SAPhysicalPackageDiskPhysicalPackage (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | SMX_SAPhysicalPackage |
| PartComponent | SMX_SADiskPhysicalPackage |

### SMX_SAPhysicalPackageStorageEnclosure (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | SMX_SAPhysicalPackage |
| PartComponent | SMX_SAStorageEnclosure |

### SMX_SADriveCageDiskPhysicalPackage (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | SMX_SADriveCage |
| PartComponent | SMX_SADiskPhysicalPackage |

### SMX_SAStoragePoolStorageExtent (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | SMX_SAStoragePool |
| PartComponent | SMX_SAStorageExtent |

### SMX_SAPrimordialPoolStorageExtent (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | SMX_SAPrimordialPool |
| PartComponent | SMX_SAStorageExtent |

### SMX_SAStorageSpecificCollectionArraySystem (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Collection | SMX_SAStorageSpecificCollection |
| Member | SMX_SAArraySystem |

### SMX_SAArrayControllerRedundancySet (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Collection | SMX_SARedundancySet |
| Member | SMX_SAArrayController |

### SMX_SAStorageRedundancySetStorageExtent (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Collection | SMX_SAStorageRedundancySet |
| Member | SMX_SAStorageExtent |

### SMX_SAArraySystemFirmware (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| InstalledSoftware | SMX_SAFirmware |
| System | SMX_SAArraySystem |

### SMX_SASCSIProtocolControllerStorageVolume (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | SMX_SASCSIProtocolController |
| Dependent | SMX_SAStorageVolume |
| DeviceAccess | 2 (Read Write) |
| DeviceNumber | Device LUN number |

### SMX_SAFirmwareArraySystem (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | SMX_SAFirmware |
| Dependent | SMX_SAArraySystem |
| ElementSoftwareStatus | ElementSoftwareStatus[0]: 2 (Current) ElementSoftwareStatus[1]: 6 (Installed) |

### SMX_SAStorageEnclosureProcessorEnclosureFirmware (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | SMX_SAEnclosureFirmware |
| Dependent | SMX_SAStorageEnclosureProcessor |
| ElementSoftwareStatus | ElementSoftwareStatus[0]: 2 (Current) ElementSoftwareStatus[1]: 6 (Installed) |

### SMX_SADiskDriveDiskDriveFirmware (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | SMX_SADiskDriveFirmware |
| Dependent | SMX_SADiskDrive |
| ElementSoftwareStatus | ElementSoftwareStatus[0]: 2 (Current) ElementSoftwareStatus[1]: 6 (Installed) |

### SMX_SADiskDriveStorageExtent (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | SMX_SADiskDrive |
| Dependent | SMX_SAStorageExtent |

### SMX_SADiskPhysicalPackageDiskDrive (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | SMX_SADiskPhysicalPackage |
| Dependent | SMX_SADiskDrive |

### SMX_SAStoragePoolStorageVolume (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | SMX_SAStoragePool |
| Dependent | SMX_SAStorageVolume |
| SpaceConsumed | Bytes consumed by volume |

### SMX_SAPrimordialPoolStoragePool (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | SMX_SAPrimordialPool |
| Dependent | SMX_SAStoragePool |
| SpaceConsumed | Bytes consumed by storage pool |

### SMX_SASpareExtentStorageExtent (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | SMX_SAStorageExtent |
| Dependent | SMX_SAStorageExtent |

### SMX_SASpareExtentStorageRedundancySet (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | SMX_SAStorageExtent |
| Dependent | SMX_SAStorageRedundancySet |

### SMX_SAStorageExtentStorageVolume (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | SMX_SAStorageExtent |
| Dependent | SMX_SAStorageVolume |

### SMX_SADAPortSCSIProtocolEndpoint (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | SMX_SADAPort |
| Dependent | SMX_SASCSIProtocolEndpoint |

### SMX_SASPIPortSCSIProtocolEndpoint (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | SMX_SASPIPort |
| Dependent | SMX_SASCSIProtocolEndpoint |

### SMX_SAStorageGroupHostedCollection (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | HP_ComputerSystem |
| Dependent | SMX_SAStorageSpecificCollection |

### SMX_SAPhysicalPackageArraySystem (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | SMX_SAPhysicalPackage |
| Dependent | SMX_SAArraySystem |

### SMX_SAArrayControllerPhysicalPackageArrayController (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | SMX_SAArrayControllerPhysicalPackage |
| Dependent | SMX_SAArrayController |

### SMX_SADiskDriveMediaAccessStatData (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| ManagedElement | SMX_SADiskDrive |
| Stats | SMX_SAMediaAccessStatData |

### SMX_SAArrayControllerPhysicalPackageArrayControllerLocation (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Element | SMX_SAArrayControllerPhysicalPackage |
| PhysicalLocation | SMX_SAArrayControllerLocation |

### SMX_SAPhysicalPackageArraySystemLocation (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Element | SMX_SAPhysicalPackage |
| PhysicalLocation | SMX_SAArraySystemLocation |

### SMX_SADriveCageDriveCageLocation (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Element | SMX_SADriveCage |
| PhysicalLocation | SMX_SADriveCageLocation |

### SMX_SADiskPhysicalPackageDiskDriveLocation (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Element | SMX_SADiskPhysicalPackage |
| PhysicalLocation | SMX_SADiskDriveLocation |

### SMX_SASCSIProtocolEndpointSCSIProtocolController (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| AvailableSAP | SMX_SASCSIProtocolEndpoint |
| ManagedElement | SMX_SASCSIProtocolController |

### SMX_SAStorageEnclosureEnclosureLocation (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Element | SMX_SAStorageEnclosure |
| PhysicalLocation | SMX_SAStorageEnclosureLocation |

### SMX_TSASSCSIProtocolEndpoint

SMX_TSASSCSIProtocolEndpoint implements the HPSA_TSASSCSIProtocolEndPoint class and represents each port on SAS Drive. The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Protocol Endpoint for <xxx> Where <xxx> is the value populated in the ElementName property of the instance of SMX_TSASPort associated to this instance via the association SMX_TSASPortTSASSCSIPE |
| Caption | Protocol Endpoint for <xxx> Where <xxx> is the value populated in the ElementName property of the instance of SMX_TSASPort associated to this instance via the association SMX_TSASPortTSASSCSIPE |
| CIM_ManagedSystemElement |  |
| Name | SMX DiskDrive.DeviceID:<unique value> Where <SMX DiskDrive.DeviceID> is the DeviceID property of the instance of SMX_DiskDrive associated to this instance via the association SMX_DiskDriveTSASSCSIPE, and unique value is a unique value across all instances of SMX_TSASSCSIProtocolEndpoint. For example, a dual port disk has two instances of SMX_TSASSCSIProtocolEndpoint with the name properties set to:- PH77MW3991:0:0 - PH77MW3991:0:1 |
| CIM_ServiceAccessPoint |  |
| SystemCreationClassName | SMX ArraySystem |
| SystemName | The property value of HP_ArraySystem.nam |
| CreationClassName | SMX TSASSCSIProtocolEndpoint |
| CIM_EnabledLogicalElement |  |
| EnabledDefault | 2 (Enabled) |
| RequestedState | 12 (Not Applicable) |
| CIM_ SCSIProtocolEndpoint |  |
| ProtocolIFType | 1 (Other) |
| OtherTypeDescription | SCSI |
| Role | 3 (Target) |
| HPSA_TSASSCSIProtocolEndpoint |  |
| Path | Redundancy path status: 0 (Unknown)2 (Active)3 (Passive)5 (Path Error) |

### SMX_ISASSCSIProtocolEndpoint

SMX_ISASSCSIProtocolEndpoint implements HPSA_ISASSCSIProtocolEndpoint and HPSA_ISASSCSIProtocolEndpoint represents each port on the smart array controller. The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Protocol Endpoint for <xxx> Where: <xxx> is the value populated in the ElementName property of the instance of SMX_TSASPort associated to this instance via the association SMX_TSASPortTSASSCSIPE |
| ElementName | Protocol Endpoint for <xxx> Where: <xxx> is the value populated in the ElementName property of the instance of SMX_TSASPort associated to this instance via the association SMX_TSASPortTSASSCSIPE |
| CIM_ManagedSystemElement |  |
| Name | The same value as the DeviceID property of the instance of SMX_ISASPort associated to this instance through the association SMX_ISASPortISASSCSIPE. |
| CIM_ServiceAccessPoint |  |
| SystemCreationClassName | SMX ArraySystem |
| SystemName | The property value of HPSA_ArraySystem.name |
| CreationClassName | SMX TSASSCSIProtocolEndpoint |
| ProtocolIFType | 1 (other) |
| OtherTypeDescription | SCSI |
| Role | 2 (initiator) |

### SMX_TSASPort

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Port <unique_index> Where: <unique_index> is a unique index across all ports on a single drive |
| ElementName | Port <unique_index> Where: <unique_index> is a unique index across all ports on a single drive |
| CIM_ManagedSystemElement |  |
| OperationalStatus | 2 (OK) For multi-path supported firmware implementations, the status of the port is OK, and the status of the path is the only element that can be detected. |
| CIM_EnabledLogicalElement |  |
| EnabledDefault | 2 (Enabled) |
| EnabledState | 5 (Not Applicable) |
| RequestedState | 12 (Not Applicable) |
| CIM_LogicalDevice |  |
| SystemnCreationClassName | SMX ArraySystem |
| SystemName | The property value of HP_ArraySystem.name |
| CreationClassName | SMX_TSASPort |
| DeviceID | The same value as the SMX_TSASSCSIProtocolEndpoint.Name property of the instance associated through SMX_ISASPortISASSCSIPE. |
| CIM_LogicalPort |  |
| UsageRestriction | 4 (Not restricted) |
| PortType | 94 (SAS) |
| CIM_SASPort |  |
| PermanentAddress | Not supported |

### SMX_ISASPort

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Port <port #> Where: <port #> is the port number For example: 1E, 2E, and so on |
| ElementName | Port <port #> Where: <port #> is the port number For example: 1E, 2E, and so on |
| CIM_ManagedSystemElement |  |
| OperationalStatus | 2 (OK) For multi-path supported firmware implementations, the status of the port is OK, and the status of the path is the only thing that can be detected. |
| CIM_EnabledLogicalElement |  |
| EnabledDefault | 2 (Enabled) |
| RequestedState | 12 (Not Applicable) |
| CIM_LogicalDevice |  |
| SystemnCreationClassName | SMX ArraySystem |
| SystemName | The property value of HP_ArraySystem.name |
| CreationClassName | SMX_ISASPort |
| DeviceID | SMX PortController.DeviceIDport # Where: SMX_PortController.deviceID is the DeviceID property of the instance of SMX_PortController associated to this instance via the association SMX_PortControllerISASPort, and port # is the port number for the port. This is the same value that is concatenated to the caption and related properties. An example of a array controller with two external ports is: PH77MW3991:1E PH77MW3991:2E |
| CIM_LogicalPort |  |
| UsageRestriction | 4 (Not restricted) |
| PortType | 94 (SAS) |
| CIM_SASPort |  |
| PermanentAddress | Not supported |

### SMX_ISASPortISASSCSIProtocolEndPoint (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | SMX_ISASPort |
| Dependent | SMX_ISASSCSIProtocolEndpoint |

### SMX_ArraySystemISASSCSIProtocolEndPoint (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | SMX_ArraySystem |
| Dependent | SMX_ISASSCSIProtocolEndpoint |

### SMX_ArraySystemISASPort (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | SMX_ArraySystem |
| PartComponent | SMX_ISASPort |

### SMX_PortControllerISASPort (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | SMX_PortController |
| Dependent | SMX_ISASPort |

### SMX_ArraySystemTSASSCSIProtocolEndpoint (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | SMX_ArraySystem |
| Dependent | SMX_TSASSCSIProtocolEndpoint |

### SMX_TSASPortTSASSCSIProtocolEndpoint (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | SMX_TSASPort |
| Dependent | SMX_TSASSCSIProtocolEndpoint |

### SMX_ArraySystemTSASPort (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | SMX_ArraySystem |
| PartComponent | SMX_TSASPort |

### SMX_DiskDriveTSASSCSIProtocolEndpoint (Association)

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| AvailableSAP | SMX_TSASSCSIProtocolEndpoint |
| ManagedElement | SMX_DiskDrive |

### SMX_SCSIInitiatorTargetLogicalUnitPath

The following table lists the associations implemented for this class.

| Property Name | Property Implementation |
| --- | --- |
| Initiator | SMX_ISASSCSIProtocolEndpoint |
| Target | SMX_TSASSCSIProtocolEndpoint |

## Provider Indications

| Indications Generated by the Provider | The following tables describe the SMX WBEM Smart Array Provider indications that are implemented for HP ProLiant server platforms where available. |
| --- | --- |

### HP_DeviceIndication: Physical Drive Status Changed to OK

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 2 (Informational) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A physical drive status has changed from a non-OK state to OK. |
| AlertingManagedElement | WBEM Path of SMX_SADiskDrive class |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 201 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | No action is recommended. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Physical Drive status OK |
| EventCategory | 6 (Primary Storage) |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | Physical Drive Status Changed to OK |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name if, one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWFirmwareVersion | Physical Disk firmware version string |
| HWLogicalLocation | Physical Disk Location string For example: Port:1E Box: 1 Bay 5 |
| HWPhysicalLocation | Physical Location of the Failed Drive (Bay Number) |
| HWSerialNumber | Physical Disk Serial Number |
| HP_DeviceIndication |  |
| DeviceModel | Physical Disk Model string |
| DeviceControllerLogicalLocation | Controller Location For example: SmartArray 642 in Slot 2 |
| VariableNames[] | FailureDescription DriveInterface LastFailureDescription |
| VariableTypes[] | String (1) uint16 (4) |
| VariableValues[] | FailureDescription – short description of the failure Drive interface HPSA_DiskDrive.DriveInterface LastFailureDescription – last failure reason string from the drive. HPSA_DiskDrive.OperationalStatus[1] |

### HP_DeviceIndication: Physical Drive Status Changed to ERROR

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | Severity 5 (Major) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A physical drive status has changed to Error. |
| AlertingManagedElement | WBEM Path of SMX_SADiskDrive class |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 202 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array ( |
| RecommendedActions | Check for failed or removed physical drives. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Physical drive status Error |
| EventCategory | 6 (Primary Storage) |
| ProbableCause | 86 (Disk Failure) |
| ProbableCauseDescription | Physical Drive Status Changed to Error |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWFirmwareVersion | Physical Disk firmware version string |
| HWLogicalLocation | Physical Disk Location string |
|  | For example: Port:1E Box: 1 Bay 5 |
| HWPhysicalLocation | Physical Location of the Failed Drive (Bay Number) |
| HWSerialNumber | Physical Disk Serial Number |
| VariableNames[] | FailureDescription DriveInterface LastFailureDescription |
| VariableTypes[] | String (1) uint16 (4) |
| VariableValues[] | FailureDescription – short description of the failure Drive interface HPSA_DiskDrive.DriveInterface LastFailureDescription – last failure reason string from the drive. HPSA_DiskDrive.OperationalStatus[1] |
| HP_DeviceIndication |  |
| DeviceModel | Physical Disk Model string |
| DeviceControllerLogicalLocation | Controller Location For example: SmartArray 642 in Slot 2 |

### HP_DeviceIndication: Physical Drive Status Changed to REBUILDING

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 2 (Informational) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A physical drive status has changed to Rebuilding. Data is now being rebuilt onto this drive from the RAID set. |
| AlertingManagedElement | WBEM Path of SMX_SADiskDrive class |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 203 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array ( |
| RecommendedActions | No action is recommended. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Physical drive is now rebuilding. |
| EventCategory | 6 (Primary Storage) |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | Physical Drive Status Changed to Rebuilding |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWFirmwareVersion | Physical Disk firmware version string |
| HWLogicalLocation | Physical Disk Location string For example: Port:1E Box: 1 Bay 5 |
| HWPhysicalLocation | Physical Location of the Failed Drive (Bay Number) |
| HWSerialNumber | Physical Disk Serial Number |
| VariableNames[] | FailureDescription DriveInterface LastFailureDescription |
| VariableTypes[] | String (1) uint16 (4) |
| VariableValues[] | FailureDescription – short description of the failure Drive interface HPSA_DiskDrive.DriveInterface LastFailureDescription – last failure reason string from the drive. HPSA_DiskDrive.OperationalStatus[1] |
| HP_DeviceIndication |  |
| DeviceModel | Physical Disk Model string |
| DeviceControllerLogicalLocation | Controller Location For example: SmartArray 642 in Slot 2 |

### HP_DeviceIndication: Physical Drive Status Changed to PREDICTIVE FAILURE

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 5 (Major) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A physical drive status has changed to Predictive Failure. |
| AlertingManagedElement | WBEM Path of SMX_SADiskDrive class |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 204 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array ( |
| RecommendedActions | Replace drive immediately to avoid loss of data. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Physical drive status Predictive Failure. |
| EventCategory | 6 (Primary Storage) |
| ProbableCause | 86 (Disk Failure) |
| ProbableCauseDescription | Physical Drive Status Changed to Predictive Failure. |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name if one exists |
| RackUID * | Rack Unique Identifier if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWFirmwareVersion | Physical Disk firmware version string |
| HWLogicalLocation | Physical Disk Location string For example: Port:1E Box: 1 Bay 5 |
| HWPhysicalLocation | Physical Location of the Failed Drive (Bay Number) |
| HWSerialNumber | Physical Disk Serial Number |
| VariableNames[] | FailureDescription DriveInterface LastFailureDescription |
| VariableTypes[] | String (1) uint16 (4) |
| VariableValues[] | FailureDescription – short description of the failure Drive interface HPSA_DiskDrive.DriveInterface LastFailureDescription – last failure reason string from the drive. HPSA_DiskDrive.OperationalStatus[1] |
| HP_DeviceIndication |  |
| DeviceModel | Physical Disk Model string |
| DeviceControllerLogicalLocation | Controller Location For example: SmartArray 642 in Slot 2 |

### HPSA_StorageVolumeIndication : Logical Drive Status Changed to OK

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 2 (Informational) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A logical drive status has changed from a non-OK state to OK. |
| AlertingManagedElement | WBEM Path of SMX_SAStorageVolume Class |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 101 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | No action is recommended. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Logical drive status OK |
| EventCategory | 6 (Primary Storage) |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | Logical Drive Status Changed to OK |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWLogicalLocation | Logical drive number and RAID Level For example: LogicalDrive1 (RAID 1+0). |
| VariableNames[] | FailureDescription |
| VariableTypes[] | String (1) |
| VariableValues[] | FailureDescription short description of the failure |
| HP_DeviceIndication |  |
| HPSA_StorageVolumeIndication |  |
| DeviceOperationalStatus | Current OperationalStatus values for Storage Volume DeviceOperationalStatus[0]: SMX_SAStorageVolume.OperationalStatus[0] DeviceOperationalStatus[1]: SMX_SAStorageVolume.OperationalStatus[1] (This index is only populated if there is additional Vendor unique status available) |
| DeviceOperationalStatusPrevious | Previous OperationalStatus values for Storage Volume DeviceOperationalStatusPrevious[0]: SMX_SAStorageVolume.OperationalStatus[0] DeviceOperationalStatusPrevious[1]: SMX_SAStorageVolume.OperationalStatus[1] (This index is only populated if there is additional Vendor unique status available) |

### SMX_SAStorageVolumeIndication: Logical Drive Status Changed to ERROR

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 5 (Major) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A logical drive status has changed to Error. |
| AlertingManagedElement | WBEM Path of SMX_SAStorageVolume class |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 102 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | Check for failed or removed physical drives. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Logical drive status error |
| EventCategory | 6 (Primary Storage) |
| ProbableCause | 86 (Disk Failure) |
| ProbableCauseDescription | Logical Drive Status Changed to Error |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWLogicalLocation | Logical drive number & RAID Level. For example: LogicalDrive1 (RAID 1+0). |
| VariableNames[] | FailureDescription |
| VariableTypes[] | String (1) |
| VariableValues[] | FailureDescription short description of the failure |
| HP_DeviceIndication |  |
| SMX_SAStorageVolumeIndication |  |
| DeviceOperationalStatus | Current OperationStatus values for Storage Volume DeviceOperationalStatus[0]: SMX_SAStorageVolume.OperationalStatus[0] DeviceOperationalStatus[1]: SMX_SAStorageVolume.OperationalStatus[1] (This index is only populated if there is additional Vendor unique status available) |
| DeviceOperationalStatusPrevious | Previous OperationStatus values for Storage Volume DeviceOperationalStatusPrevious[0]: SMX_SAStorageVolume.OperationalStatus[0] DeviceOperationalStatusPrevious[1]: SMX_SAStorageVolume.OperationalStatus[1] (This index is only populated if there is additional Vendor unique status available) |

### SMX_SAStorageVolumeIndication: Logical Drive Status Changed to Degraded

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 2 (Info) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A logical drive is in a degraded state. Device is still servicing I/O but redundancy may be degraded or lost. |
| AlertingManagedElement | WBEM Path of SMX_SAStorageVolume class |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 103 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | Check status of physical drives. Degraded status may be caused by failed or missing physical drives, or a data recovery operation on one or more physical drives. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Logical drive in degraded state |
| EventCategory | 6 (Primary Storage) |
| ProbableCause | 86 (Disk Failure) |
| ProbableCauseDescription | Logical State Changed to Degraded |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWLogicalLocation | Logical drive number & RAID Level. For example: LogicalDrive 1 (RAID 1+0). |
| VariableNames[] | FailureDescription |
| VariableTypes[] | String (1) |
| VariableValues[] | FailureDescription short description of the failure |
| HP_DeviceIndication |  |
| SMX_SAStorageVolumeIndication |  |
| DeviceOperationalStatus | Current OperationStatus values for Storage Volume DeviceOperationalStatus[0]: SMX_SAStorageVolume.OperationalStatus[0] DeviceOperationalStatus[1]: SMX_SAStorageVolume.OperationalStatus[1] (This index is only populated if there is additional Vendor unique status available) |
| DeviceOperationalStatusPrevious | Previous OperationStatus values for Storage Volume DeviceOperationalStatusPrevious[0]: SMX_SAStorageVolume.OperationalStatus[0] DeviceOperationalStatusPrevious[1]: SMX_SAStorageVolume.OperationalStatus[1] (This index is only populated if there is additional Vendor unique status available) |

### HP_DeviceIndication: Controller Status Changed to OK

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 2 (Info) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A Smart Array controller status has changed from non-OK to OK. |
| AlertingManagedElement | WBEM Path to SMX_SAArrayController class. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 1 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | No action is recommended. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Controller status OK |
| EventCategory | 6 (Primary Storage) |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | Array Controller Status Changed to OK |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWFirmwareVersion | Array Controller firmware version string |
| HWLogicalLocation | Array Controller Location string For example: Smart Array P600 in Slot 5 |
| HWManufacturer | Array Controller Manufacturer |
| HWSerialNumber | Array Controller Serial Number |
| VariableNames[] | FailureDescription CacheSizeTotal CacheSerialNumber |
| VariableTypes[] | String (1) uint32 (5) |
| VariableValues[] | FailureDescription – short description of the failure CacheSizeTotal HPSA_ArrayController.CacheSizeTotal CacheSerialNumber HPSA_ArrayController.CacheSerialNumber |
| HP_DeviceIndication |  |
| DeviceModel | Array Controller Model string |

### HP_DeviceIndication: Controller Status Changed to ERROR

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 5 (Major) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A Smart Array controller status has changed to Error. |
| AlertingManagedElement | WBEM Path to SMX_SAArrayController class. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 2 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array ( |
| RecommendedActions | Check controller status and run diagnostics. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Controller status Error |
| EventCategory | 6 (Primary Storage) |
| ProbableCause | 24 (I/O device error) |
| ProbableCauseDescription | Array Controller Status Changed to Error |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWFirmwareVersion | Array Controller firmware version string |
| HWLogicalLocation | Array Controller Location string For example: Smart Array P600 in Slot 5 |
| HWManufacturer | Array Controller Manufacturer |
| HWSerialNumber | Array Controller Serial Number |
| VariableNames[] | FailureDescription CacheSizeTotal CacheSerialNumber |
| VariableTypes[] | String (1) uint32 (5) |
| VariableValues[] | FailureDescription – short description of the failure CacheSizeTotal HPSA_ArrayController.CacheSizeTotal CacheSerialNumber HPSA_ArrayController.CacheSerialNumber |
| HP_DeviceIndication |  |
| DeviceModel | Array Controller Model string |

### HP_DeviceIndication: Controller Redundancy state has been set to Active

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 3 (Warning) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A Smart Array controller redundancy state has changed from Standby to Active. |
| AlertingManagedElement | WBEM Path to SMX_SAArrayController class. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 3 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | Check status of all controllers. An array controller may have failed causing this controller to become active. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Controller redundancy state set to Active |
| EventCategory | 6 (Primary Storage) |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | Array Controller Redundancy State Changed to Active |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name if one exists |
| RackUID * | Rack Unique Identifier if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWFirmwareVersion | Array Controller firmware version string |
| HWLogicalLocation | Array Controller Location string For example: Smart Array P600 in Slot 5 |
| HWManufacturer | SAEnclosureFirmware |
| HWSerialNumber | Array Controller Serial Number |
| VariableNames[] | FailureDescription CacheSizeTotal CacheSerialNumber |
| VariableTypes[] | String (1) uint32 (5) |
| VariableValues[] | FailureDescription – short description of the failure CacheSizeTotal HPSA_ArrayController.CacheSizeTotal CacheSerialNumber HPSA_ArrayController.CacheSerialNumber |
| HP_DeviceIndication |  |
| DeviceModel | Array Controller Model string |

### HP_DeviceIndication: Controller Redundancy has been lost

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 3 (Warning) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A Smart Array controller has detected that redundancy has been lost due to failure or loss of communication with partner controller. |
| AlertingManagedElement | WBEM Path to SMX_SAArrayController class. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 4 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | Check status of the partner array controller. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Controller redundancy lost |
| EventCategory | 6 (Primary Storage) |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | Array Controller Redundancy has been Lost |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWFirmwareVersion | Array Controller firmware version string |
| HWLogicalLocation | Array Controller Location string For example: Smart Array P600 in Slot 5 |
| HWManufacturer | Array Controller Manufacturer |
| HWSerialNumber | Serial Number |
| VariableNames[] | FailureDescription CacheSizeTotal CacheSerialNumber |
| VariableTypes[] | String (1) uint32 (5) |
| VariableValues[] | FailureDescription – short description of the failure CacheSizeTotal HPSA_ArrayController.CacheSizeTotal CacheSerialNumber HPSA_ArrayController.CacheSerialNumber |
| HP_DeviceIndication |  |
| DeviceModel | Array Controller Model string |

### HP_DeviceIndication: Controller Redundancy has been restored

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 2 (Info) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | Controller has detected that Redundancy has been restored and can communicate with partner controller. |
| AlertingManagedElement | WBEM Path to SMX_SAArrayController class. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 5 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | No action is recommended. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Controller redundancy restored |
| EventCategory | 6 (Primary Storage) |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | Array Controller Redundancy has been Restored |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName* | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWFirmwareVersion | Array Controller firmware version string |
| HWLogicalLocation | Array Controller Location string For example: Smart Array P600 in Slot 5 |
| HWManufacturer | Array Controller Manufacturer |
| HWSerialNumber | Array Controller Serial Number |
| VariableNames[] | FailureDescription CacheSizeTotal CacheSerialNumber |
| VariableTypes[] | String (1) uint32 (5) |
| VariableValues[] | FailureDescription – short description of the failure CacheSizeTotal HPSA_ArrayController.CacheSizeTotal CacheSerialNumber HPSA_ArrayController.CacheSerialNumber |
| HP_DeviceIndication |  |
| DeviceModel | Array Controller Model string |

### HP_DeviceIndication: Controller Battery Status Change - OK

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 2 (info) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A Smart Array controller Battery status has changed from non-OK to OK. |
| AlertingManagedElement | WBEM Path to SMX_SAArrayController class. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 6 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | No action is recommended. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClass | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Controller battery status OK |
| EventCategory | 6 (Primary Storage) |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | Controller Battery Status Changed to OK |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWFirmwareVersion | Array Controller firmware version string |
| HWLogicalLocation | Array Controller Location string For example: Smart Array P600 in Slot 5 |
| HWManufacturer | Array Controller Manufacturer |
| HWSerialNumber | Array Controller Serial Number |
| VariableNames[] | FailureDescription CacheSizeTotal CacheSerialNumber |
| VariableTypes[] | String (1) uint32 (5) |
| VariableValues[] | FailureDescription – short description of the failure CacheSizeTotal HPSA_ArrayController.CacheSizeTotal CacheSerialNumber HPSA_ArrayController.CacheSerialNumber |
| HP_DeviceIndication |  |
| DeviceModel | Array Controller Model string |

### HP_DeviceIndication: Controller Battery Status Changed to not fully charged

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 3 (Warning) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A Smart Array controller Battery status has changed to Not Fully Charged. |
| AlertingManagedElement | WBEM Path to SMX_SAArrayController class. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 7 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | Check battery operation, allow time to the battery to be fully charged. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Controller Battery Status set to Not fully Charged. |
| EventCategory | 6 (Primary Storage) |
| ProbableCause | 99 (low battery) |
| ProbableCauseDescription | Controller Battery Status Changed to Not Fully Charged |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWFirmwareVersion | Array Controller firmware version string |
| HWLogicalLocation | Array Controller Location string |
|  | For example: Smart Array P600 in Slot 5 |
| HWManufacturer | Array Controller Manufacturer |
| HWSerialNumber | Array Controller Serial Number |
| VariableNames[] | FailureDescription CacheSizeTotal CacheSerialNumber |
| VariableTypes[] | String (1) uint32 (5) |
| VariableValues[] | FailureDescription – short description of the failure CacheSizeTotal HPSA_ArrayController.CacheSizeTotal CacheSerialNumber HPSA_ArrayController.CacheSerialNumber |
| HP_DeviceIndication |  |
| DeviceModel | Array Controller Model string |

### HP_DeviceIndication: Controller Battery Status Changed to FAILED

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 5 (major) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A Smart Array controller battery status changed to Failed. |
| AlertingManagedElement | WBEM Path to SMX_SAArrayController class. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 8 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | Check battery connection and replace if necessary. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Controller battery status Failed |
| EventCategory | 6 (Primary Storage) |
| ProbableCause | 92 (battery failure) |
| ProbableCauseDescription | Controller Battery Status Changed to Failed |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWFirmwareVersion | Array Controller firmware version string |
| HWLogicalLocation | Array Controller Location string For example: Smart Array P600 in Slot 5 |
| HWManufacturer | Array Controller Manufacturer |
| HWSerialNumber | Array Controller Serial Number |
| VariableNames[] | FailureDescription CacheSizeTotal CacheSerialNumber |
| VariableTypes[] | String (1) uint32 (5) |
| VariableValues[] | FailureDescription – short description of the failure CacheSizeTotal HPSA_ArrayController.CacheSizeTotal CacheSerialNumber HPSA_ArrayController.CacheSerialNumber |
| HP_DeviceIndication |  |
| DeviceModel | Array Controller Model string |

### HP_DeviceIndication: Controller Cache Status - OK

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 2 (Info) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A Smart Array controller Cache status has changed from non-OK to OK. |
| AlertingManagedElement | WBEM Path to SMX_SAArrayController class. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 9 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | No action is recommended. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Controller cache status OK |
| EventCategory | 6 (Primary Storage) |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | Controller Cache Status Changed to OK |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWFirmwareVersion | Array Controller firmware version string |
| HWLogicalLocation | Array Controller Location string |
|  | For example: Smart Array P600 in Slot 5 |
| HWManufacturer | Array Controller Manufacturer |
| HWSerialNumber | Array Controller Serial Number |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name if one exists |
| RackUID * | Rack Unique Identifier if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemGUID | Platform GUID of the computer system generating the indication |
| VariableNames[] | FailureDescription CacheSizeTotal CacheSerialNumber |
| VariableTypes[] | String (1) uint32 (5) |
| VariableValues[] | FailureDescription – short description of the failure CacheSizeTotal HPSA_ArrayController.CacheSizeTotal CacheSerialNumber HPSA_ArrayController.CacheSerialNumber |
| HP_DeviceIndication |  |
| DeviceModel | Array Controller Model string |

### HP_DeviceIndication: Controller Cache Status - Temporarily Disabled

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 2 (Info) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A Smart Array controller Cache status has changed to Temporarily Disabled. This may be caused by a rebuild or configuration change. Normal operation will return after background operations have completed. |
| AlertingManagedElement | WBEM Path to SMX_SAArrayController class. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 10 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | No action is recommended. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Array controller cache status temporarily disabled |
| EventCategory | 6 (Primary Storage) |
| ProbableCause | 35 (Performance Degraded) |
| ProbableCauseDescription | Array Controller Cache Status Changed to Temporarily Disabled |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWFirmwareVersion | Array Controller firmware version string |
| HWLogicalLocation | Array Controller Location string For example: Smart Array P600 in Slot 5 |
| HWManufacturer | Array Controller Manufacturer |
| HWSerialNumber | Array Controller Serial Number |
| VariableNames[] | FailureDescription CacheSizeTotal CacheSerialNumber |
| VariableTypes[] | String (1) uint32 (5) |
| VariableValues[] | FailureDescription – short description of the failure CacheSizeTotal HPSA_ArrayController.CacheSizeTotal CacheSerialNumber HPSA_ArrayController.CacheSerialNumber |
| HP_DeviceIndication |  |
| DeviceModel | Array Controller Model string |

### HP_DeviceIndication: Controller Cache Status - Permanently Disabled

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 3 (Warning) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A Smart Array controller Cache status has changed to Permanently Disabled. The controller has detected a problem with the cache and has disabled the cache permanently. |
| AlertingManagedElement | WBEM Path to SMX_SAArrayController class. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 11 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | Check the cache controller error code and run diagnostics. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Array controller cache status permanently disabled |
| EventCategory | 6 (Primary Storage) |
| ProbableCause | 35 (Performance Degraded) |
| ProbableCauseDescription | Array Controller Cache Status Changed to Permanently Disabled |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWFirmwareVersion | Array Controller firmware version string |
| HWLogicalLocation | Array Controller Location string For example: Smart Array P600 in Slot 5 |
| HWManufacturer | Array Controller Manufacturer |
| HWSerialNumber | Array Controller Serial Number |
| VariableNames[] | FailureDescription CacheSizeTotal CacheSerialNumber |
| VariableTypes[] | String (1) uint32 (5) |
| VariableValues[] | FailureDescription – short description of the failure CacheSizeTotal HPSA_ArrayController.CacheSizeTotal CacheSerialNumber HPSA_ArrayController.CacheSerialNumber |
| HP_DeviceIndication |  |
| DeviceModel | Array Controller Model string |

### HP_DeviceIndication: Enclosure Power Supply Status changed to Non Redundant

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 3 (Warning) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A Storage Enclosure Power Subsystem status has changed to non-redundant. |
| AlertingManagedElement | WBEM Path of SMX_SAStorageEnclosureProcessor |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 301 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | Check status of all enclosure power supplies. Ensure that power supplies have not failed. Add or replace power supplies as necessary. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Power Supply status Non-Redundant |
| EventCategory | 17 (Primary Storage Power) |
| ProbableCause | 89 (Power Supply Failure) |
| ProbableCauseDescription | Redundant Power Supply Status Changed to Non-Redundant |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWLogicalLocation | Enclosure Location string For example: Port 1I Box 1 |
| HWSerialNumber | Enclosure Serial Number |
| HP_DeviceIndication |  |
| DeviceModel | Enclosure Model string |

### HP_DeviceIndication: Enclosure Power Supply Status changed to OK (Redundant)

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 2 (Info) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A Storage Enclosure Power Subsystem has changed to Fully Redundant. |
| AlertingManagedElement | WBEM Path of SMX_SAStorageEnclosureProcessor |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 302 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | No action is recommended. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Redundant power supply status OK |
| EventCategory | 17 (Primary Storage Power) |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | Redundant Power Supply Status Changed to OK |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name if one exists |
| RackUID * | Rack Unique Identifier if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWLogicalLocation | Enclosure Location string For example: Port 1I Box 1 |
| HWSerialNumber | Enclosure Serial Number |
| HP_DeviceIndication |  |
| DeviceModel | Enclosure Model string |

### HP_DeviceIndication: Enclosure Power Supply Status changed to Redundant Degraded

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 3 (Warning) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A Storage Enclosure Power Subsystem status has changed to Degraded Redundancy. |
| AlertingManagedElement | WBEM Path of SMX_SAStorageEnclosureProcessor |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 303 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | Check status of all enclosure power supplies. Ensure that power supplies have not failed. Add or replace power supplies as necessary. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Power supply status Redundant Degraded |
| EventCategory | 17 (Primary Storage Power) |
| ProbableCause | 89 (Power supply failure) |
| ProbableCauseDescription | Redundant Power Supply Degraded |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWLogicalLocation | Enclosure Location string For example: Port 1I Box 1 |
| HWSerialNumber | Enclosure Serial Number |
| HP_DeviceIndication |  |
| DeviceModel | Enclosure Model string |

### HP_DeviceIndication: Enclosure Temperature sensor Status changed to OK

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 2 (Info) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A Storage Enclosure Temperature sensor status changed from non-OK to OK. |
| AlertingManagedElement | WBEM Path of SMX_SAStorageEnclosureProcessor |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 307 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array ( |
| RecommendedActions | No action is recommended. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Temperature sensor status OK |
| EventCategory | 24 (Primary Storage Cooling) |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | Temperature Status Changed to OK |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWLogicalLocation | Enclosure Location string For example: Port 1I Box 1 |
| HWSerialNumber | Enclosure Serial Number |
| HP_DeviceIndication |  |
| DeviceModel | Enclosure Model string |

### HP_DeviceIndication: Enclosure Temperature sensor Status Changed to Warning

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 3 (Warning) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A Storage Enclosure Temperature sensor status changed to Warning Condition. This indicates the enclosure temperature is above normal, otherwise enclosure is still operating normally. |
| AlertingManagedElement | WBEM Path of SMX_SAStorageEnclosureProcessor |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 308 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | Ensure that the system is adequately ventilated. Check for proper room temperature and internal/external airflow. Add or repair fans and air baffling if necessary. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Temperature sensor status Warning Condition |
| EventCategory | 24 (Primary Storage Cooling) |
| ProbableCause | 51 (Temperature Unacceptable) |
| ProbableCauseDescription | Temperature Status Changed to Warning Condition |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWLogicalLocation | Enclosure Location string For example: Port 1I Box 1 |
| HWSerialNumber | Enclosure Serial Number |
| HP_DeviceIndication |  |
| DeviceModel | Enclosure Model string |

### HP_DeviceIndication: Enclosure Temperature sensor Status Changed to Critical

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 5 (major) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A Storage Enclosure Temperature sensor status has changed to Critical. An enclosure temperature is reaching the maximum allowed temperature for normal operation. A shutdown is imminent. Action is required before loss of data occurs. |
| AlertingManagedElement | WBEM Path of SMX_SAStorageEnclosureProcessor |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 309 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | Ensure that the system is adequately ventilated. Check for proper room temperature and internal/external airflow. Add or repair fans and air baffling if necessary. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Temperature sensor status Critical |
| EventCategory | 24 (Primary Storage Cooling) |
| ProbableCause | 51 (Temperature Unacceptable ) |
| ProbableCauseDescription | Temperature Status Changed to Critical |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWLogicalLocation | Enclosure Location string For example: Port 1I Box 1 |
| HWSerialNumber | Enclosure Serial Number |
| HP_DeviceIndication |  |
| DeviceModel | Enclosure Model string |

### HP_DeviceIndication: Enclosure Fan Status Changed to OK

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 2 (Info) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A Storage Enclosure Fan status has changed from a non-OK state to OK. |
| AlertingManagedElement | WBEM Path of SMX_SAStorageEnclosureProcessor |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 304 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | No action is recommended. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Fan status OK |
| EventCategory | 24 (Primary Storage Cooling) |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | Fan Status Changed to OK |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWLogicalLocation | Enclosure Location string For example: Port 1I Box 1 |
| HWSerialNumber | Enclosure Serial Number |
| HP_DeviceIndication |  |
| DeviceModel | Enclosure Model string |

### HP_DeviceIndication: Enclosure Fan Status changed to Degraded

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 3 (Warning) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A Storage Enclosure Fan status has changed to Degraded. An enclosure has detected there may a problem with one or more of the fans. |
| AlertingManagedElement | WBEM Path of SMX_SAStorageEnclosureProcessor |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 305 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | Ensure that the system is adequately ventilated. Check for proper room temperature and internal/external airflow. Add or repair fans and air baffling if necessary. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Fan status Degraded |
| EventCategory | 24 (Primary Storage Cooling) |
| ProbableCause | 94 (Fan failure) |
| ProbableCauseDescription | Fan Degraded |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWLogicalLocation | Enclosure Location string For example: Port 1I Box 1 |
| HWSerialNumber | Enclosure Serial Number |
| HP_DeviceIndication |  |
| DeviceModel | Enclosure Model string |

### HP_DeviceIndication: Enclosure Fan Status change to Failure

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication |
| PerceivedSeverity | 5 (major) |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| Description | A Storage Enclosure Fan status changed to Failure. An enclosure has detected a failure in the fan subsystem and there is insufficient fan capability to ensure normal operation. |
| AlertingManagedElement | WBEM Path of SMX_SAStorageEnclosureProcessor |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 306 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Smart Array |
| RecommendedActions | Ensure that the system is adequately ventilated. Check for proper room temperature and internal/external airflow. Add or repair fans and air baffling if necessary. |
| SystemName | Name of the computer system generating the indication |
| SystemCreationClassName | CreationClassName of the computer system generating the indication |
| HP_AlertIndication |  |
| Summary | Fan status Error |
| EventCategory | 24 (Primary Storage Cooling) |
| ProbableCause | 94 (Fan failure) |
| ProbableCauseDescription | Fan Failure |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| ProviderVersion | For example: 2.2.0.0 |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_HardwareIndication |  |
| HWLogicalLocation | Enclosure Location string For example: Port 1I Box 1 |
| HWSerialNumber | Enclosure Serial Number |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties that follow and are indicated with an asterisk (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists |
| RackUID * | Rack Unique Identifier, if one exists |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment |
| HP_DeviceIndication |  |
| DeviceModel | Enclosure Model string |

## Physical Location

The Physical Location is a string representing the Smart Array device physical location. This string should represent the physical location of the device with which an end-user can uniquely locate the device. Most of these strings will be represented in customer documentation, silkscreen labels, or hood tags.

The following table lists the properties implemented. Any combination of the following applicable descriptors could be used to better define the device location.

| All Systems | HP Integrity Cellular Servers | HP BladeServers in C3000 / C7000 Enclosures |
| --- | --- | --- |
| Slot=<num> | Cabinet=<num> | Blade=<num> |
| Embedded Controller=<logical_controller_id> | Bay=<num> | RootPort=<num> (if embedded on System Board) |
| System Mainboard (referring to System Board/motherboard) | Chassis=<num> | Mezzanine=<num> (I/O Mezzanines lot num) |