==========================================================================

Callback and dial-in for Linux

------------------------------

Version 2.00

or:

A Multi-Channel Callback and Dial-In Monitoring system

using setserial 2.10 and getty_ps 2.0.7b

or higher.

Frank B. Brokken

e-mail: frank@icce.rug.nl

==========================================================================

original cbstat program by

Karel Kubat

(karel@icce.rug.nl)

Contents overview

-----------------

BEFORE READING THIS ALL

INTRODUCTION

Copyright

Obsolete versions and programs

Obtaining the callback program

PREREQUISITES

The modem

The modem under Linux

Modem registers

tornado 14k4, internal modem

trust 14k4, external modem

Dial-in under Linux

getty_ps

/etc/inittab and symbolic links

/etc/default/getty.ttyS?

MANUAL CALLBACK

THE CALLBACK PROGRAMS

The Program cb

modem-lines

command-abbreviation

dial-in mode

callback mode

some examples

The Program cblogin

The states of cblogin

getty definition files

Other files

CALLBACK.SETUP

INSTALLING CALLBACK

TROUBLE SHOOTING

==============================================================================

BEFORE READING THIS ALL

-----------------------

This is not the quick-installation guide for the callback system, but

rather an extensive description of what callback is and how it works.

If you're primarily interested in how to install callback on your computer,

continue with reading the file INSTALL.

If you're a previous user of the callback system (i.e., versions before 2.00),

please read the CHANGELOG file as well, especially the tail of it:  starting

with version 2.00 the organization of the callback setup has changed

considerably.

INTRODUCTION

------------

There are basically two reasons for wanting a callback system.

- The first of these reasons may be that you wish to allow call-in on your

Linux box, but want to enhance the security of the call-in system by enforcing

a phone back scheme from your Linux box to one of a set of recognized

telephone numbers:  this effectively allows a modem login only from `known

friendly' phones.

- The other reason is of course the cost of telephone connections. Since a

login session by callback originates from a Linux box (equiped with a dial-out

modem), the cost of the connection will be charged to the number to which the

dial-out compuuter is connected (this situation may not be representative for

countries all over the world, but at least it holds true for Europe, where, as

we know, Linux originated).

Apart from that, a dial-in monitoring system comes in handy in cases where

- You and your computer share the telephone line: you might not want your

computer to answer the phone in cases where you are being called.

- Your computer is being called at regular times from another computer,

using the callout-facility: you might not always want to answer that

incoming telephone call.

The setup of the callback/dial-in scheme described below, is briefly as

follows:

(a) A remote machine dials into a Linux box and requests a phone back

connection to be made.

(b) The login process at the Linux box performs some checking (e.g.,

validates the user and validates the requested phone back number) and

breaks the connection.

(c) Subsequently, the Linux box dials the requested phone back number.

At this point, the user of the remote machine should have hung up the

phone and should be waiting for a ring.  The remote machine now should

answer the phone and connect.

(d) Using this second connection, originating at the Linux box, a full

login procedure is started.

Copyright

---------

The program callback is distributed in source format under the rules of the

Gnu Public License (GPL). The GPL is included in the distribution in the file

COPYING. Cb and its associated programs and sources is copyrighted by us

(FBB/KK).

You are therefore not allowed to modify the program without our prior consent.

The reason for this is that we'd like to have the `last version' of callback;

We hate encountering `our' programs with a version like 9.13 while we're still

running 2.00.

If you have suggestions for improvements of the program, bug reports, or

comments on the documentation or otherwise, please mail us.  We can be

reached at:

Frank B. Brokken, ICCE          e-mail: frank@icce.rug.nl

phone:  (+31) 50 63 36 88

Westerhaven 16

9718 AW  Groningen

The Netherlands

(Note that Karel's e-mail and phone number is not listed here: this year

he has other obligations, so please do not flood his mailbox with

questions/comments about callback.

Obsolete versions and programs

------------------------------

Previously distributed versions of

- callback (using the cbstat program)

- dim (the dial-in monitor)

are now considered obsolete, and are no more supported by us.

Obtaining the callback program

------------------------------

The callback package, which is described in this document, is available from

our primary ftp-site is contained in an archive callback-X.YY.tar.gz, where

X.YY is the version number.  This file is available as callback.doc and is

also contained in the archive.

The callback package is also available at the Linux distribution sites

tsx-11.mit.edu (in pub/linux/sources/usr.bin/Communications)

and

sunsite.unc.edu (in pub/Linux/system/Serial).

PREREQUISITES

-------------

There is a number of prerequisites before the callback scheme can be

implemented.

The modem

---------

First of all, the Linux box must be equiped with a modem capable of answering

the phone and of dialling out.  If you don't have a modem yet for your Linux

system, but are considering buying one, go for a Hayes compatible modem.  The

remainder of this document shows examples for this type of modem.

Second, the modem must of course be connected to a telephone. Make sure that

the modem functions properly: it may be a good idea to boot under DOS and

try to talk to your modem via a communications package.  E.g., supposing

that you have MSKERMIT (the MSDOS version of Kermit), try booting DOS,

starting mskermit, and typing:

set port com<X>         {this sets the COM2 (external modem) or COM4

(internal modem) port}

{alternatively, use:

set port com<X> <port-address> <irq-nr>}

set baud <speed>        {this sets the modem speed}

c                       {this should connect to the modem}

at                      {this sends an attention signal to the modem}

OK                      {modem should respond: OK}

atdt 1234567            {this instructs your modem to dial (atd) using

tones (t) the phone number 1234567,

your modem should start dialling,

use atdp for pulse dialling}

NO RESPONSE or BUSY     {modem response if the phone number is

or NO CARRIER            invalid or does not connect to a modem}

or

CONNECT <speed>         {modem response if the phone number leads to

a modem}

The modem under Linux

---------------------

The modem must function properly under Linux. The suggested modem device

names are:

/dev/ttyS1 and /dev/cua1        for an external modem, and

/dev/ttyS2 and /dev/cua2        for an internal modem.

You may need to create these devices, if you don't have them yet. The details

on this are in the SERIAL.FAQ.  In the remainder of this document, /dev/ttyS1,

/dev/ttyS2 and /dev/ttyS3 will be used in the examples; You may need to

recompile your kernel with serial IO support.  Furthermore, you may have to

use the program setserial, version 2.10 or higher (ftp-able from sunsite's

pub/Linux/system/Serial directory).  The setserial program takes many

parameters; such as the uart type of the modem.

E.g., using the setup used in recent slackware distributions:

${SETSERIAL} /dev/cua0 uart 16450 port 0x3F8 irq 4

${SETSERIAL} /dev/cua1 uart 16450 port 0x2F8 irq 3

${SETSERIAL} /dev/cua2 uart 16450 port 0x3E8 irq 5

could be necessary.  These define the uart of the modem. In order to use the

irq 5, you may have to select it using a dipswitch; That number is often a

good one to select if your other serial lines are busy serviceing a mouse and

a serially connected terminal.  (originally, it's the irq number of the second

parallel port).

After configuring the modem for Linux, making sure that your kernel supports

serial IO, etc., try talking to it through kermit.  Use the above session

example to test the working.  Try calling out to a `real' system and test

the reliability of the connection.

Modem registers:

----------------

Setting modem registers to their right values can be a very trying experience.

Below some examples of modem-registersettings are given which have been shown

to work with callback. Note that the registersettings of the modems is not

as such part of the callback system. Examples are provided to get you started.

Realize the information is given here as an example, not as the ultimate

answer: some experimenting may still be neccessary, unfortunately.

tornado 14k4, internal modem

----------------------------

ACTIVE PROFILE:

B1 E1 L0 M1 N1 Q0 T V1 W2 X4 Y0 &C1 &D3 &G0 &J0 &K0 &Q6 &R0 &S0 &T4 &X0 &Y0

S00:000 S01:000 S02:043 S03:013 S04:010 S05:008 S06:002 S07:050 S08:002 S09:006

S10:014 S11:095 S12:050 S18:000 S25:005 S26:001 S36:007 S37:000 S38:020 S44:003

S46:138 S48:007 S49:008 S50:255

STORED PROFILE 0:

B1 E1 L0 M1 N1 Q0 T V1 W2 X4 Y0 &C1 &D3 &G0 &J0 &K0 &Q6 &R0 &S0 &T4 &X0

S00:000 S02:043 S06:002 S07:050 S08:002 S09:006 S10:014 S11:095 S12:050 S18:000

S25:005 S26:001 S36:007 S37:000 S38:020 S44:003 S46:138 S48:007 S49:008 S50:255

STORED PROFILE 1:

B1 E1 L0 M1 N1 Q0 T V1 W2 X4 Y0 &C1 &D3 &G0 &J0 &K0 &Q6 &R0 &S0 &T4 &X0

S00:000 S02:043 S06:002 S07:050 S08:002 S09:006 S10:014 S11:095 S12:050 S18:000

S25:005 S26:001 S36:007 S37:000 S38:020 S44:003 S46:138 S48:007 S49:008 S50:255

TELEPHONE NUMBERS:

0=                                      1=

2=                                      3=

OK

trust 14k4, external modem

--------------------------

Here are the register-settings of the Trust 14k4 external modem:

ACTIVE PROFILE:

B1 E1 L0 M1 N1 Q0 T V1 W2 X4 Y0 &C1 &D3 &G0 &J0 &K0 &Q6 &R0 &S0 &T4 &X0 &Y0

S00:000 S01:000 S02:043 S03:013 S04:010 S05:008 S06:002 S07:050 S08:002 S09:006

S10:014 S11:095 S12:050 S18:000 S25:005 S26:001 S36:007 S37:000 S38:020 S44:003

S46:138 S48:007 S49:008 S50:255

STORED PROFILE 0:

B1 E1 L0 M1 N1 Q0 T V1 W2 X4 Y0 &C1 &D3 &G0 &J0 &K0 &Q6 &R0 &S0 &T4 &X0

S00:000 S02:043 S06:002 S07:050 S08:002 S09:006 S10:014 S11:095 S12:050 S18:000

S25:005 S26:001 S36:007 S37:000 S38:020 S44:003 S46:138 S48:007 S49:008 S50:255

STORED PROFILE 1:

B1 E1 L0 M1 N1 Q0 T V1 W2 X4 Y0 &C1 &D3 &G0 &J0 &K0 &Q6 &R0 &S0 &T4 &X0

S00:000 S02:043 S06:002 S07:050 S08:002 S09:006 S10:014 S11:095 S12:050 S18:000

S25:005 S26:001 S36:007 S37:000 S38:020 S44:003 S46:138 S48:007 S49:008 S50:255

TELEPHONE NUMBERS:

0=                                      1=

2=                                      3=

OK

Dial-in under Linux

-------------------

Before you attempt to construct the callback procedure, you must make sure

that dialing in to your Linux system works.  For the dial-in you need the

appropriate getty or uugetty: for this we use getty_ps version 2.0.7b (or

higher).  The sources for getty_ps can be obtained at, e.g., tsx-11.mit.edu.

When compiled, the sources yield two new programs getty and uugetty, which can

be used instead of the default (uu)getty. The difference between getty and

uugetty is that uugetty monitors the activity on the modem line using lock

files in /usr/spool/uucp, while getty monitors the activity only according to

internal modem flags (busy/not busy).  According to the documentation, getty

should be able to detect modem activity, hence making uugetty obsolete.

Whether getty or uugetty is used in your setup is irrelevant in respect to the

callback scheme; We will use uugetty or getty equivalently in the following

description. Recent developments suggest that as far as the modem setup is

concerned, both programs getty and uugetty work.

getty_ps

--------

(Not part of callback, but used to watch the modem-line)

When constructing the new (uu)getty programs, follow the instructions closely.

Make sure NOT do do a "make install" right away: first, do a "make" and test

the new (uu)getty as suggested on only one virtual console.  A "make install"

with a non-working uugetty will overwrite the previous (working) version,

thereby causing you to cut your own jugular (i.e., fubarring your system).

Once you have the new getty and uugetty working and installed as

etc/(uu)getty, adapt the following files to get dial-in working:

/etc/inittab, /etc/gettydefs, /etc/default/uugetty.ttyS* (the

modifications for these files are described in the getty_ps documentation

and are illustrated below.

/etc/inittab and symbolic links

-------------------------------

Here is a sample from the file inittab (see also the directory examples):

s1:5:respawn:/sbin/modem.ttyS1 ttyS1 14400

s2:5:respawn:/sbin/modem.ttyS2 ttyS2 14400

Don't be alarmed at the 'modem.ttyS?' parts: the current version of

callback supports multiple modem-lines, and the cb-program should be informed

what process will be connected to what modemline. Assuming that /sbin/getty_ps

is being used, symbolic links must have been set up in order to allow init

to respawn modem*. E.g., the command

ln -s /sbin/getty_ps /sbin/modem.ttyS1

must have been given in order to use the modem hooked up on the serial line

/dev/ttyS1 (usually the serial port, to which an external modem is connected).

For the other modemline the analogous

ln -s /sbin/getty_ps /sbin/modem.ttyS2

would be required.

The respawn lines instruct init, the first program which is run under Linux, to

execute /sbin/modem.ttyS1 (or 2, or both, depending on what you tell

init through inittab) upon startup.

The actual program, getty_ps, will now continuously watch /dev/ttyS{1,2} for

activity.

When activity is detected, getty_ps will try to establish a connection

according to two files: /etc/gettydefs and the corresponding

/etc/default/getty.ttyS{1,2}.  The specification "respawn" causes init to

restart the uugetty process when it dies or when its subprocesses die.  The

argument "14400" is a label for the file /etc/gettydefs.

/etc/gettydefs

The file /etc/gettydefs defines the type of connection to establish. Here is

a part of /etc/gettydefs:

14400 # B19200 CS8 # B19200 SANE -ISTRIP\

#\nThis is modem port @L of @S.\n\nPlease login: #\

9600

9600 # B9600 CS8 # B9600 SANE -ISTRIP\

#\nThis is modem port @L of @S.\n\nPlease login: #\

4800

4800 # B4800 CS8 # B4800 SANE -ISTRIP\

#\nThis is modem port @L of @S.\n\nPlease login: #\

2400

2400 # B2400 CS8 # B2400 SANE -ISTRIP\

#\nThis is modem port @L of @S.\n\nPlease login: #\

14400

Once getty detects activity on the specified input line, it will search for

the label "14400" (as mentioned in /etc/inittab) in its definition file

/etc/gettydefs.  That label occurs here, and instructs getty to try to

establish a 14400 baud connection. If the other party uses a slower modem,

an mutually acceptable speed should be negociated:

Getty will loop through gettydefs searching for entries of the "next label"

type.  In my case, the "next" field of the "14400" entry points to the line

starting with the label "9600": getty will retry a 9600 baud connection.

Should that fail too, getty will use the "next" label of 4800: which is the

line starting with "4800", etcetera. The last line, which tries for a 1200

baud connection, points back to the "14400" line. Please note that the empty

lines between the specifications are necessary.

/etc/default/getty.ttyS?

------------------------

In order to tell getty what to do, a /etc/default/getty.ttyS* file is needed

for every modem line that is used. Below the getty.ttyS1 file is given as an

example. If you use ttyS2 (too), the file getty.ttyS2 file must be available

as well.

The file /etc/default/getty.ttyS* is used by the suggested getty program.

Here is an example of the file /etc/default/getty.ttyS1:

SYSTEM=your.system.name

VERSION=/proc/version

ISSUE=/conf/callback/callback.iss

LOGIN=/conf/callback/cblogin

HANGUP=YES

INIT="" ATH\r OK ATZ\r OK ATS0=1\r OK

TIMEOUT=120

CONNECT=""

Here's an alternative, used by a (slow) 2400 baud modem:

SYSTEM=your.system.name

VERSION=/proc/version

LOGIN=/bin/login

ISSUE=/etc/issue

CLEAR=NO

HANGUP=YES

INIT="" ATH\r OK ATZ\r OK ATS0=1&C1&D3\r OK

TIMEOUT=120

CONNECT=""

These files define the following actions, to be performed by getty once a

connection is established:

(a) The first four lines define text information, to be displayed during

the login process.  Refer to the getty_ps information for the meaning of

these lines; The line LOGIN=/bin/login defines the program which

is used to log in a user, the line LOGIN=/conf/callback/cblogin does the

same, but the cblogin (described below) program is used for the

callback-procedure.

(b) The lines CLEAR=NO and HANGUP=YES define the way that getty

handles the modem. Refer to the getty_ps information.

(c) The line

INIT="" ATH\r OK ATZ\r OK ATS0=1\r OK

is one of getty_ps' new and all-powerful features. This line consists of

strings, which are in turn either expected by getty to be read from

the modem line, or which are sent to the modem.  The INIT label tells

getty to execute this sequence upon startup; i.e., when first

accessing the modem.

The sequence can be read as follows:

- Getty should wait for no special string on the modem line ("").

- Getty should then send the string "ATH", followed by a

carriage-return, to the modem. This hangs up the phone line.

- Getty should then wait for the string OK to appear on the modem

line; that is the modem's response.

- Getty should then send "ATZ", followed by a carriage-return. This

re-initializes the modem.

- Getty should then wait for an OK.

- Next, the string "ATS0=1\r" is sent to the modem. This

string consists of a number of flags, which program a Hayes

compatible modem to perform certain actions. Refer to your

modem documentation for a similar setup. E.g., the modem is

instructed to pick up the phone after 1 ring.

Note the string used for Karel's modem:

INIT="" ATH\r OK ATZ\r OK ATS0=1&C1&D3\r OK

Please note that the AT commands, which are sent to the modem, apply to a

`Hayes-compatible' modem.  We have no idea in howfar this refers to true

compatibility, but it works around here.  Be sure to check your modem

documentation for the exact syntax applying to your modem when using these

commands for your own setup.

(d) The TIMEOUT specification instructs getty to break the connection

when the user waits longer than 120 seconds before logging in.

(e) The line

CONNECT=""

causes getty to wait until a connection comes in. Again, please refer

to the getty_ps documentation.

After installing getty_ps and modifying all necessary files, you should have

the dial-in protocol established.  Test the dialling in before attempting

the next step: manual phone back.

MANUAL CALLBACK

---------------

A callback scheme which is maintained `by hand' is fairly easy to establish.

To comprehend how this scheme works, an understanding of the login process

is necessary.

The initial program of the Linux operating system, init, spawns (uu)getty

processes to watch for activity on all incoming lines: virtual consoles,

modem lines, etc.  Once (uu)getty sees activity on one of these input lines

over which it watches, it interprets the corresponding definitions file in

/etc/default (e.g., /etc/default/getty.ttyS2) and performs all appropriate

actions, such as the waiting for a connection, the answering of the phone.

If all succeeds, (uu)getty performs a number of actions: first, the standard

input, output and error streams are remapped to the line which (uu)getty is

watching.  Then (uu)getty executes the login program which is given in the

definition file (see the line LOGIN=/bin/login in the second sample

/etc/getty.ttS2 above).  This means that getty itself is overlayed with the

login program which in turn inherits the standard input, output and error

streams.  The login program therefore does not `know' from which line it

receives its input: the actual input may come from the keyboard, from a modem,

from a serial null modem or from whichever source.  As far as the login

program is concerned, all input comes from the standard input stream, all

output goes to the standard output stream, etc..

The login program may then perform a number of actions: validating the

user's password, starting a shell, etc.  Once the connection is broken

(e.g., when the user's shell terminates), the control returns to init: the

original starter of getty.  Init then re-spawns getty to watch over the same

line (the respawning is in fact defined in /etc/inittab using the

specification respawn).

With this information at hand, getty can be coaxed into dialling a given

telephone number to establish a modem connection.  E.g., say you wanted your

Linux system to dial number 5551234, you could install the following file as

/etc/default/getty.ttyS2:

LOGIN=/conf/callback/cblogin

ISSUE=/conf/callback/callback.iss

HANGUP=YES

INIT="" ATH\r OK ATZ\r OK ATDT5551234\r CONNECT\s\A

TIMEOUT=600

CONNECT=""

or, used on the slow (2400) modem:

LOGIN=/conf/callback/cblogin

ISSUE=/etc/issue

CLEAR=NO

HANGUP=YES

INIT="" ATH\r OK ATZ\r OK ATD5551234\r

TIMEOUT=60

CONNECT=CONNECT\s\A

This definition instructs getty to perform the following modem action upon

startup (this is the INIT command):

a. Getty waits for no special information on the modem line ("").

b. Then, the phone is hung op (ATH\r). The string OK must appear on

the modem line.

c. After this, the modem is re-initialized (ATZ\r). Again, OK must appear.

d. Now, getty sends ATD5551234\r to the modem. Instead of ATD the explicit

ATDT or ATDP may be necessary. The ATDT command causes

the modem to dial a number using tone dialling, ATDP uses pulse

dialling.

e. After this, getty waits for the string CONNECT, followed by a space

and followed by a number, to appear on the modem line. The number is

used for auto-bauding.

Following the INIT specification, getty remaps its input and output streams

and executes the login program. Keyboard (i.e., input) actions must have

been sensed within the timeout-interval. If nothing happens within this

interval, getty will disconnect.

The resulting action is the following: when the definitions file is first

read, which is upon startup of the getty process, the number 5551234 is

dialled.  When a connection is made, the login program is executed.  The

person at the other end of the line, at the phone with the number 5551234,

therefore hears the phone ring and can answer it by using a modem.  The

linux login prompt then appears.

Should that person not have a modem, getty will not be able to establish a

connection and will exit.  Init, seeing that its spawned process getty died,

will respawn getty.  This getty will again ring 5551234, ad infinitum.

Installing a `forever-ringing' getty definition may be an effective way to

drive your enemy crazy, but it is not the idea of a slick callback system.

However, you might want to try to establish a similar manual `phone forever'

connection before continuing to the next section, in order to get a feel of

the processes involved.  To test this setup, follow

a. Make sure that dial-out and dial-in are both working properly. This

indicates that getty and the modem can communicate.

b. Edit the file /etc/default/getty.ttyS2 according to the dial-out

example above.  Preferably choose a number where a modem is connected,

so that you may see if a real login session is established.

c. Do a "kill -1" to all processes which watch the modem. You can get a

list of these processes using "ps -ax". E.g., say your file /etc/inittab

contains the fillowing startup specification of /etc/getty:

s2:5:respawn:/sbin/modem.ttyS2 ttyS2 14400

When getty processes are running, the output of ps -ax should show

a line like:

11190  ?  S     0:00 /sbin/modem.ttyS2 ttyS2 14400

meaning that a process with the number 11190 is watching over the modem.

In this case, you should do a "kill -1 11190". Thanks to the special name

"modem.ttyS?", which we suggest to use, you (but also the

cb-program, described below) can quickly determine the process which is

watching the modem using

ps -ax | grep modem.ttyS2

The file "/etc/modem.ttyS2" is actually a link to /etc/getty and can

be created with

ln -s /etc/getty /etc/modem.ttyS2

If you don't have any matching getty processes (yet), you obviously do

not have to kill them.

d. Then, do a "kill -1 1", this restarts the program with process ID 1.

This is always init.  The flag -1 causes kill to send a `re-initialize'

signal: init will not die, but will restart all its spawned processes.

e. Due to the restarting of init, a new getty is spawned which watches

over /dev/ttyS3.  This getty should now read the definitions file and

should start dialling.

THE CALLBACK PROGRAMS

---------------------

Three programs which are provided:

- The program cb is the callback/dial-in monitoring program, controlled by

user commands.

- The program cblogin is the program monitoring (callback) login-sessions.

- The program cbgetty is a shell around getty, monitoring the number of

callback retries, restricting the number of dial-outs to a maximum.

When a callback-system is installed, all three programs are necessary. When a

dial-in managing system is installed, only cb is necessary. All programs

should be accessible and executable by root only.

The Program cb

--------------

If you used Karel Kubat's original cbstat program before, then the largest

modifications are encountered here. The cb program operates in one of two

modes:

1. The mode `callback' is set in cases where the callback-software

must monitor both dial-in and dial-out (callback) situations.

2. The mode 'dial-in' is set in cases where the callback software

must monitor only dial-in situations.

The two modes are mutually exclusive, and are set via a configuration file

(callback.setup), described below.

The program displays (to stdout) usage information when invoked without

any arguments. The usage information is mode-dependent.

Cb shares the major part of its code and it shares callback.setup with

callback's login program cblogin. Cb itself is normally invoked with

arguments. The arguments form a command, and possibly define a modem-line.

modem-lines

-----------

Since the callback software recognizes a default modemline, a final argument

may be given to cb to indicate a particular modemline. A modemline can be

specified as follows:

ttyS1-ttyS9

1-9

+ (i.e., all configured lines).

The modemline is ignored with the 'init'-command. When specified, it is always

the last argument to cb.

command-abbreviation

--------------------

Commands to cb may be abbreviated, e.g., "in" is equivalent to  "init".

dial-in mode

------------

In the dial-in mode cb can be given the following commands:

disable:	to disables dial-in

enable:		to enable dial-in

file:		to shows the actual definition file

init:		to restart init

E.g., a command like

cb enable

can be given to enable dial-ins on the thefault modem line.

callback mode

-------------

If cb is operating in the callback-mode, extra commands are possible, above

and beyond those available with the dial-in mode. The extra commands are:

list [who]:	to list user-related information

state:		to show the  state of callback

who where:	to call (force a callback to) 'who' at 'where'

some examples

-------------

cb init		- restart init

cb en 1		- enable modemline ttyS1

cb f +		- show all active getty-files

cb lis john	- list all callback info about user `john'

cb mary funny	- call the callback destination `funny', allowing

mary to connect to the computer running cb.

The Program cblogin

-------------------

The cblogin program is used with the callback-mode. It is not used for the

dial-in mode. Cblogin is a simple tool which is run by getty as a bogus login

program.  Depending on the callback state, cblogin places two versions of the

file /etc/default/getty.ttyS? in /etc/default:  one standard version (used for

dialing-in) and a version (used for dialing-out). cblogin also performs

authentication on dial-in: only certain users are allowed to pass the callback

gate, either to realize an actual callback, or to login to the computer

directly. Alternatively, when activated after a dial-out, it passes the

previously mentioned username to the login-program. This is both a service to

the user granted access to the callback-system, as a simple security device,

as the previously provided username is now the default value passed to the

login program.

It is important to note that the program cblogin does NOT perform any user

logins. The program handling the actual login is /bin/login. cblogin is nothing

but a shell around /bin/login.

The states of cblogin

---------------------

The actions of cblogin are defined by a state, kept in a state file. Callback

has two states.

State 0 (zero) is the initial state, in which cblogin expects an incoming

call. The corresponding getty definitions file is the following (note that

these may not be the settings which we actually use, refer to the examples

directory for up-to-date example files):

SYSTEM=your.computer.system

VERSION=/proc/version

ISSUE=/conf/callback/callback.iss

LOGIN=/conf/callback/cblogin

HANGUP=YES

INIT="" ATH\r OK ATZ\r OK ATS0=1\r OK

TIMEOUT=120

CONNECT=""

Calling the line where this getty-configuration file is active, results in the

following actions:

1. When an incoming call occurs, getty executes the program

/conf/callback/cblogin.

2. The cblogin program now validates the user, and asks for a

callback-destination.

3. If all information is valid, cblogin prepares for state one:

- a new definitions file /etc/default/getty.ttyS* is created,

- a new statefile for state 1 is created,

- cblogin exits.

4. The initial dial-in connection is broken.

5. Init restarts getty.

Due to the respawning of getty, getty now reads the configuration file that is

waiting for it (this is state 1). This file will contain a callback number,

for example:

LOGIN=/conf/callback/cblogin

ISSUE=/conf/callback/callback.iss

HANGUP=YES

INIT="" ATH\r OK ATZ\r OK ATDT5551234\r CONNECT\s\A

TIMEOUT=600

CONNECT=""

In this situation the modem will start to dial number 5551234. The modem at

the other end of the line answers the incoming call. Cblogin now:

1. Restores the getty configuration file corresponding to state 0,

2. Resets the state file to state 0,

3. overlays itself with the /bin/login program, providing it with a

default usename: the name of the user originally authenticated for

callback.

getty definition files

----------------------

Cblogin and cb both use getty definition files. These files are located in a

particular directory, e.g., /conf/callback, and are, respectively:

1. For enabling (dial-in) purposes:		    enable.ttyS*

2. Fore callback purposes:			    callback.ttyS*

3. For disabling purposes (only used by cb):    disable.ttyS*

Note the `*' in ttyS*: for each modemline this set of files must be

available. So, if you use line ttyS1 as well as ttyS3, the files

/conf/callback.enable.ttyS1

/conf/callback.enable.ttyS3

/conf/callback.callback.ttyS1

/conf/callback.callback.ttyS3

/conf/callback.disable.ttyS1

and

/conf/callback.disable.ttyS3

must be available.

Since the cb and cblogin programs will have to enter a particular phonenumber

in the getty configuration file used for callback, the callback.ttyS? files

have the following general form:

LOGIN=/conf/callback/cblogin

ISSUE=/etc/issue

CLEAR=NO

HANGUP=YES

INIT="" ATH\r OK ATZ\r OK ATD$PHONE\r

TIMEOUT=60

CONNECT=CONNECT\s\A

In these files the string $PHONE must occur at the place where the phonenumber

to call back must be inserted.  This marker, $PHONE, must occur literally and

in upper case.  The programs cb and cblogin will replace this marker with the

requested phone number.

Samples of all the configuration files can also be found in the examples/

directory: this directory may contain more recent examples than the ones shown

in this documentation.

Other files

-----------

Two other files are relevant for the operation of cblogin: state.ttyS* and

callback.iss.

The file /conf/callback/state.ttyS*.st is the file in which cblogin

maintains the state of the line ttyS*.  When this file is absent or when its

first character is a '0', cblogin assumes that it is operating in state 0

(ready to accept incoming calls).  Otherwise, cblogin assumes state one. This

file is entirely maintained by cblogin: there is no need to create the file or

change its contents manually. The statefile also contains a second number,

which is not actually used in state 0, representing the user-id of the user

who is expected to login after being called back. This user-id is not the uid

as found in, e.g., /etc/passwd, but refers to the ordering of the user in the

callback.setup file (descussed below).

The file /conf/callback/callback.iss is an empty file, which is displayed in

place of /etc/issue by the callback login. This file is not strictly

necessary: /etc/issue would suffice, though some text would be displayed

during login. The issue file is a good place to put a warning message like

"You must be authorized to use this access line" and such.

CALLBACK.SETUP

--------------

The most important difference between versions 1.xx and 2.xx of callback is

the way the software is told what to accept and what to reject. Starting with

version 2.00, practically everything you may want to configure is defined in

the file callback.setup. What's *not* configurable are the places the callback

software looks for the file callback.setup (unless, of course, you adapt the

sources).

The software looks for callback.setup in the following locations, stopping

the search whenever a file 'callback.setup' is found:

1.	/etc/callback.setup

2.	/usr/etc/callback.setup

3.  /usr/local/etc/callback.setup

The callback.setup is best installed without any access rights to anybody but

root, since it contains security-sensitive information. E.g., do a

chmod 600 callback.setup

as root.

The program cb can be configured in one of two modes: dial-in, in which mode cb

will only be used to enable and disable the modem, and callback, in which mode

cb and cblogin will be used to realize the callback facilities.

Examples of callback.setup files for these two modes are found in the

examples/ subdirectory: dial-in.setup shows an example of the setup file used

for with the dial-in mode, callback.setup shows an example of the setup file

used with the callback-mode.

Note, however, that whatever mode you will be using, the configuration file

looked for is *always* named callback.setup. So, in the dial-in mode the

configuration will also be found in the /etc/callback.setup file.

Please refer to the examples/ directory for further information about the

configuration file.

INSTALLING CALLBACK

-------------------

The installation of the package is described in the file INSTALL.

TROUBLE SHOOTING

----------------

Problems which may arise when using the callback scheme may be caused by an

incorrect installation. When something `just doesn't work', the program cb

should be used to check the working of the package `by hand'. The

instructions in the file INSTALL show how cb can be compiled and installed. If

you suspect an incorrect installation, please use cb to check all

installation files: if cbstat cannot properly access the information in these

files, cblogin won't either. You can use cb to imitate the callback

process: e.g., use cbstat to force the callback to a given number.

The program cblogin has no obvious bugs (yet), though it is very well possible

that bugs are present in the program.  If you find bugs, please mail me

(frank@icce.rug.nl) and describe the bug.  Or even better, hunt down the

error, find a remedy and mail me the working code which corrects the bug.

The setup of the callback strategy however does have quirks (a quirk is a

documented bug, so there you are).  E.g., the modem of the Linux box may fail

to answer.  The reason for this might be that a previous callback session was

interrupted (e.g., when the remote computer hung up the phone) while a session

bound to the modem still runs.  A remedy may be enforcing auto-logout times

after a given number of minutes of inactivity, thereby forcing the modem to

come on-line again after some time.  If you suspect this quirk, do a "ps -ax"

and watch for all processes bound to, e.g., S2 for /dev/ttyS2.  There may

exist a process -tcsh indicating a user session which is bound to S2, while no

one is using the modem.  This would typically be the result of a previous

session which was interrupted, leaving the tcsh running without a proper

connection.  In such a situation, kill the appropriate process(es); a new

getty generation should then come on-line and take over the modem. For this

reason, you might want to enforce auto-logout times after a given number of

minutes of inactivity. E.g., when you enforce auto-logout after 10 minutes of

inactivity, an interrupted session will die by itself --- thereby freeing the

way for subsequent modem usage.

A second quirk is the following. When a remote computer dials in and requests

a phone back connection, the callback connection may never come.  This can be

caused by the fact that the remote computer's phone is never hung up.  The

problem here is that cblogin has to cut the connection, hoping that the other

side will too.  If the other side maintains the connection, the telephone line

may be kept busy for a long time.  This quirk is caused by the telephone

exchange: some systems keep a line from caller A to recipient B busy until A

hangs up --- B can hang up the phone and pick it up again, the connection is

still there.  Callback therefore can never accomplish a successful

connnection, since the line is always busy.  A second remote computer then

also cannot dial in, since the phone at the Linux box is occupied.  There is

no known remedy for this quirk.  Luckily, the first caller who causes the quirk

will have a high phone bill for keeping a line busy for a long time.  That

should (eventually) teach the bastard!