IDE TRANSLATION MODE SETTINGS

The allowable IDE Translation Mode settings are:

·	Standard CHS - Standard Cylinder, Head, Sector.	This method, in use for years, provides parameters to the drive as Head, Cylinder and Sector values.

·	LBA - Logical Block Addressing. This method overcomes limitations imposed by the Int 13H BIOS interface as well as limitations imposed by Standard CHS IDE access. This method is specific to, and must be supported by the drive. It provides a 28 bit sector identifier to the drive and is based upon the ATA Specification (ANSI X3.221:1993).

·	ECHS - Extended CHS. This method can be used to overcome some of the limitations imposed by the Int 13H BIOS interface and allows access to drives with more than 1024 cylinders. This method is transparent to the drive firmware. It works by decreasing the number of logical cylinders and increasing the number of logical heads as presented to the Int 13H interface, and then having the BIOS convert these logical values back into their physical values before presenting them to the drive.

·	AUTO - Auto-configure IDE Translation Mode. This selection will choose one of the above translation methods. The first choice is to utilize LBA if it is supported by the drive. The second choice is to utilize ECHS if the drive topology allows. If neither of the above two methods are utilized, then Standard CHS is chosen.

Different operating systems have different abilities regarding IDE Translation Modes:

·	UNIX operating systems (as currently implemented) do not support either LBA or ECHS and must utilize the Standard CHS method. UNIX can support drives larger than 528MB, but does so in it's own way.

·	OS/2 2.1 can support LBA, ECHS or Standard CHS methods . Note that LBA support may require a switch setting on an OS/2 driver in order to operate in that mode. (*)

·	OS/2 1.x & Novell Netware can support either ECHS or Standard CHS methods, but not LBA.

·	DOS & Windows can support LBA, ECHS or Standard CHS methods. If the "Fast 32-bit IDE" option of Windows is utilized, make sure to use the  Standard CHS setting only. This Windows option should NOT be used if the drive supports an Enhanced IDE Access method such as PIO Mode 3 or DMA. If PIO Mode 3 is available, the BIOS will select it (there is no SETUP control associated with this feature). This is no great loss however since using such access methods will result in higher throughput than using the  "Fast 32-bit IDE" option. (*)

(*) - If the drive was formatted with custom parameters and AUTO is selected, the drive may not boot. If this occurs, use the Standard CHS translation method.